#### TM-D700 Configuration for ARISS

27 June 2003

Bob Bruninga, WB4APR, US Naval Academy

Program Memories:

This file documents the five default setup options for the D700 to be flown on ISS.  With the two hundred different radio channels the nearly one hundred TNC parameters and the variety of applications for this radio on orbit, the default configurations are absolutely critical to being able to maintain communications with the crew under all conditions.  These five configurations reduce operations to these fundamental configuration baselines:

PM1:	Voice Operations (mono band)

PM2:	Voice Operations (cross band)

PM3:	APRS/Packet and BBS operations

PM4:	Attached PC and packet operations

PM5:	Emergency Voice and alternate 9600 baud Packet Operations.

PM-off:	No defaults.  This mode is for knowledgible licensed crew member’s experimentation

The advantage of these five Program Memories (PM’s) are that they can be restored to defaults with a two-button key press by the crew at any time.  The PM’s remember the following types of parameters for the radio:

- Default Channel for LEFT side and RIGHT side of radio

- Which side of radio the Microphone and PTT will activate

- Which side of the radio the TNC will RECEIVE and on which side it will Transmit

- 36 special TNC parameters for each PM

- The function of the several “soft keys” on the radio front panel

Channel/Frequency Memories:

Notice that PM’s do not store any combination of Channel frequencies other than the initial two defaults.  This means, that once a PM has been selected by the crew, this only configures the radio to a known default pair of channels.  He can still tune to any channel after that.  Also, he can chose to switch the Microphone/PTT band as well.  Although this offers the ultimate in flexibility (millions of combinations), it also shows the value of using the five PM’s to always return the radio to a known initial state.

In addition, each of the 200 memory channels remember separate TX and RX frequencies, offsets, and PL or CTCSS tones.   The D700 is a dual radio system and although it only supports two channels at a time, it is very important to remember that each channel consists of both a displayed RECEIVE frequency and a separate TRANSMIT frequency.  Thus, at any time, there are a total of four frequencies involved in radio operations.

Since the Microphone and PTT (for voice) can be using one channel and the TNC can be using the same or the other channel, or even can transmit on one channel and receive on the other, there are many conventional and non-conventional ways to use these combinations for ARISS.

PUBLIC and COMMAND Channels:

In general, we are recommending that PUBLIC frequencies be paired in many cases with non-public command channels that can be used to contact the crew from special ARISS Command Ground stations in the background even while the radio is in use on its public frequencies.   This document attempts to configure the radio for that type of general configuration.

#### Program Memory Crew Operations Summary				             page 2  26 June 03

| PM | FUNCTION | RADIO LEFT SIDE | RADIO RIGHT SIDE | OPERATIONAL DESCRIPTION |
| --- | --- | --- | --- | --- |
| ### PM1 | ### Voice Operations Single Bands | PRIVATE ARISS CHANNEL Front Panel TEXT from ARISS command stns Spkr: Crew hears ARISS command stations | CREW SELECTS VOICE CHANNELS Spkr: Crew hears users | ### General Voice Operations (monoband, VHF/UHF) * For Schools, ARISS, and other crew contacts * No user APRS nor PACKET |
| ### PM2 | ### Voice Operations - Crossband - Repeater mode | CREW SELECTS DOWNLINKS Spkr: Crew hears ARISS command stations | CREW SELECTS UPLINKS Front Panel TEXT from command stations Spkr: Crew hears users | General Voice Operations (crossband) - (VHF,UHF,Lband) * Can set Voice REPEATER mode * No user APRS nor PACKET |
| ### PM3 | ### APRS/Packet/BBS | Packet user Downlinks | Packet user Uplinks Front Panel TEXT to and from users SPKR: Crew may talk voice to Command stations with CTCSS | Unattended APRS & packet operations, (crossband) * USERS have all APRS and Packet modes * Crew has front panel APRS Lists, Text and Messages |
| ### PM4 | ### PACKET/crewPMS | PACKET User Downlinks | Packet User Uplinks SPKR: Crew may talk voice to Command stations with CTCSS | ### Packet mode for attached PC on serial port * USERS have all Digipeat modes and BBS * Crew has no APRS front panel TEXT displays * Crew does have PC access to PMS/BBS, chat mode |
| ### PM5 | ### EMERGENCY | 9600 baud packet experimental | CREW TALKS HERE (with PL) | ### Quick access to emergency channels |

Notes on Channel Programming:  Consider these items in channel planning:

1)  TX/RX combinations:  Each channel can store independent RX, TX frequencies, PL/CTCSS tones.  But the D700 can also operate crossband with LEFT and RIGHT side, so any combination of channels can be used independently.

2) Speakers & CTCSS:   All channels not specifically used for attended crew voice should have CTCSS set to mute speaker.  We do not want the crew to change Volume controls on different channels for different modes.  Or to ever turn the volume down.  If volume is down, we loose ability for ARISS command stations to talk to or alert crew.

3)  CTCSS:  The same RF frequency can be saved in multiple channels with different PL or CTCSS settings so the crew does all changes with Channel settings only.  CTCSS can be set on receive without setting matching PL on transmit (eg 9600 Baud channel).

##### Program Memory Technical Settings Table							Page 3

| PM | FUNCTION | LEFT SIDE | RIGHT SIDE | DESCRIPTION |
| --- | --- | --- | --- | --- |
| PM1 | ### Voice - Simplex - Split Freqs Command Stn - Private Text - Private Voice | CMD-1 [D] * MYcall RS0ISS-1 * Digipeat off * No Beacons * CTCSS=XXX * APRS ptt-mode Front Panel TEXT | OPS-1 [PTT] * Default channel should have CTCSS * School channels should have Tx PL | * RIGHT side for voice operations on any channel. * LEFT side is for ARISS command stations - shows TEXT from ground command stations. - Ignores any other packets unless matching ALTNET - hears voice from command stations with XXX PL - Can send APRS text message to command station * No user APRS nor PACKET |
| PM2 | ### Voice - Crossband Command Stn - Private Text - Private Voice | FDUX-1 [Tx][PTT] * MYcall RS0ISS-2 * Digipeat off * No beacons * CTCSS=XXX * APRS ptt-mode Downlinks: V/U | FDUX-2 [Rx] Front Panel TEXT Uplinks: V/U/Lband | Same as PM1 but crossband. Crew sets Left and Right sides - All advantages of PM1 but also Full-Duplex - Can set REPEATER mode - Command stations can send TEXT on uplink w ALTNET - Command stations can TALK private PL on downlink * No user APRS nor PACKET |
| PM3 | ### APRS/PACKET - crossband Command Stn - Private Voice | APRS-1 [Tx] * MYcall RS0ISS-3 * Digipeat ON * Beacon Text * BBS is ON * CTCSS=XXX * APRS auto-mode Front Panel TEXT | APRS-2 [Rx][PTT] * CTCSS=YYY | Unattended APRS and packet operations, crossband * RIGHT side is uplink, * LEFT side is downlink * Command stations can TALK private PL on uplink * Command stations can TALK private PL on downlink * Crew can TALK down semiprivately on uplink * USERS have all packet modes: - User-to-User APRS or User-to-crew APRS - UI digipeating, Chat mode, BBS operations |
| PM4 | ### PACKET - crossband Command Stn - Private Voice Serial Port Packet (no front panel txt) | PCKT-1 [Tx] * MYcall RS0ISS-4 * Digipeat ON * Beacon Text * BBS is ON * CTCSS=XXX | PCKT-2 [Rx][PTT] * CTCSS=YYY | TNC in packet mode for attached serial port operation * USERS have all Digipeat modes and BBS * Crew has no radio APRS front panel TEXTdisplay * Attached PC may be used for crew CHAT mode * Attached PC may run special experiments or OPS modes |
| PM5 | ### EMERGENCY 9600 Packet - crossband Front Panel text | PAKET96 [Tx] * MYcall RS0ISS-5 * Digipeat ON * Beacon Text * BBS is ON * CTCSS=XXX * APRS auto-mode | EMER-1 [Rx][PTT] CREW TALKS HERE (with PL) | *RIGHT side is Emergency Voice mode * LEFT and RIGHT side can be used on different channel for experiments with 9600 baud packet using APRS and BBS and all digipeater modes. |

Page 4

Following APRS TNC parameters are common to all PM’S except where noted:

3-1  MYcall = Various.         RS0ISS-# where # is the PM#  number

3-2  GPS NMEA

3-3  Waypoint 9 digits NMEA

3-4  My Position 0000.00/00000.00

3-5  Pos Ambiguity = 4 digits (Position to nearest Degree (60 miles))

3-6  Posn Comment = Various.  (PM1,2=Off Duty, PM3=In Service, PM5=Special)

3-7  Position Limit = OFF

3-8  Station Icon= W

3-9  Status Text = Various (SEE BELOW)

3-A  Status TX rate = 1/1

3-B  Packet Path = SGATE,WIDE

3-C  Packet TX = Various     (PTT mode for voice nmodes, AUTO for others)

3-D  TX interval = 1 minute

3-E  Unprotocol = Various    (This sets the ALTNETS in PM1,2)

3-F  Beep = Mine

3-G  Mile/Kilometer = km

3-H  Temperature = Degrees C

3-I   Data Band = Various (A for PM1.  A=Tx / B=Rx for PM2,3,4,5)

3-J   Packet Speed = 1200 (for PM1-PM4, 9600 for PM5)

3-K  Digiepater = OFF for PM1,2 and ON for PM3,4,5)

3-L   UIDigi = RELAY,WIDE,ARISS,APRSAT

3-M  Auto Msg Reply = Off

3-N  Reply Message = tbd

3-O  BLN Group = none

3-P   MSG Group = ALL,QST,CQ,KWD,ISS,ARISS

The following table shows the five STATUS texts that may be included in any APRS beacon from the D700 radio.  These five different status texts may be independently selected by the crew at any time with a few key strokes.   Select APRS, STATUS-TEXT, and then use the dial to select one.  Then press the “USE” button.

Although these Status Texts are free field text , they will be appearing on thousands of radio front panels of the TM-D700 and TH-D7 walkie-talkies which have limited display screens.  Thus, with some judicious choice of text and spacing, these beacons can appear pleasantly on these small screens.

| Status | Default PM | Status Texts (max length is 37 bytes but only first 20 are visible on ALL users) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | D7 first line | D7 second line | D700 option | Visible on PC’s only |
|  |  | 1234567890 | 1234567890 | 12345678 | 901234567 |
| 1 | PM1 & 2 | ISS ALPHA_ | Ham Radio_ | ARISS |  |
| 2 |  | A Special_ | Event! |  |  |
| 3 | PM3 | APRS/PAKET | /BBS is on |  |  |
| 4 | PM5 | Greetings_ | From Space | ! |  |
| 5 |  | APRS/Paket | /BBS/SSTV | enabled. | _Enjoy! |

##### Example Table of Frequencies showing dual use of many single channels					page 5

| PM | FUNCTION | LEFT SIDE | RIGHT SIDE | OPERATIONAL DESCRIPTION |
| --- | --- | --- | --- | --- |
| ### PM1 | ### Voice Operations Single Bands | SECRET-1 [D] 14?.xxx Rx [Rx] 14?.??? Tx [Tx] CTCSS=xxx PRIVATE CHANNEL * Front Panel TEXT from command stations * Crew hears command stations using PL | OPS-1 145.800 RX 145.800 TX [PTT] CTCSS=ppp CREW SELECTS VOICE CHANNEL Spkr: Crew hears users | ### General Voice Operations (monoband, VHF/UHF) * For Schools, ARISS, and other crew contacts, crew chooses USA or European channel or School channel * No user APRS nor PACKET NOTE: “PPP” Means Public Channel or CTCSS “XXX” Means any secret frequency “YYY” Means any other secret frequency “???” Means any frequency |
| ### PM2 | ### Voice Operations - Crossband - Repeater mode | FDUX-1 144.xxx RX 145.800 TX [Tx][PTT] CTCSS=### CREW SELECTS DOWNLINKS * Crew hears command stns using PL (Responds on public downlink) | UHF-1 435.PPP RX [Rx] 437.??? TX CTCSS=ppp CREW SELECTS UPLINKS * Front Panel TEXT from cmd stations Spkr: Hears users | General Voice Operations (crossband) - (VHF,UHF,Lband) * Can set Voice REPEATER mode * No user APRS nor PACKET * This mode is mostly for future full duplex crossband operations with all user uplinks on UHF and all downlinks on VHF. But it can also do VHF/VHF or UHF/UHF pairs. |
| ### PM3 | ### APRS ### Packet and BBS | ### APRS-DN1 144.xxx Rx 145.800 Tx [Tx] CTCSS=### Crew hears secret voice Command stations with PL set. | ### APRS-UP1 145.825 Rx [Rx] 144.xxx Tx [PTT] CTCSS=### * Front Panel TEXT to and from users Crew Responds on private downlink | Unattended APRS and packet operations, (xband) * Downlinks on LEFT, Uplinks on RIGHT * USERS have all APRS and Packet modes * Crew has front panel APRS Lists, Text and Messages |
| ### PM4 | ### PACKET ### And crewPMS | ### PKT-DN1 144.xxx Rx 145.800 Tx [Tx] CTCSS=### Crew hears secret voice command stations with PL set. | ### PKT-UP1 145.990 Rx [Rx] 144.xxx Tx [PTT] Crew Responds on private downlink | ### Packet mode for serial port operation to PC * Downlinks on LEFT, Uplinks on RIGHT * USERS have all Digipeat modes and BBS * Crew has no APRS front panel TEXT displays * Crew does have PC access to PMS/BBS, chat mode |
| ### PM5 | ### EMERGENCY | ### PKT-969600 baud packet 145.800 Rx 145.800 Tx [Tx] CTCSS (only)=xxx | EMER-1 14x.xxx Rx [Rx] 14y.yyy Tx [PTT] PL=ppp CREW TALKS (PL) | ### Quick access to emergency channels And Experimental 9600 baud packet |

Page 6

APRS Channels:

# NAME	RX FREQ	TX FREQ	Description

APRS-UP1	145.825	144.???	PM3 RIGHT Side uplink (secret downlink)

APRS-UP2	145.990	144.???	Alternate PM3 RIGHT side (secret downlink)

APRS-UP3	145.800	144.???	Alternate PM3 RIGHT side (secret downlink)

APRS-DN1	144.???	145.800	PM3 LEFT Side Downlink (secret uplink)

APRS-DN2	144.???	145.825	Alternate PM3 LEFT side (secret uplink)

APRS-DN3	144.???	144.990	Alternate PM3 LEFT side (secret uplink)

APRS-ID	145.800	same		These can all be used as uplinks or

APRS-IU	145.990	same		downlinks on left or right sides in

APRS-SAT	145.825	same		any combinations

APRS-NA	144.390	same		<== North American Continent

APRS-EU	144.810	same		<== European Continent

APRS-U1	437.975	same		PCSAT2 APRS downlink (ISS Freq)

APRS-U2	435.250	same		PCsat APRS uplink

APRS-U3	435.275	same		PCSAT2 APRS downlink

APRS-GND	445.925	same		USA APRS UHF frequency

##### PACKET Channels:

In general, Packet channels are all the same as APRS channels.

Probably no need to duplicate them just for the sake of the channel NAMES.

PKT-UP	145.990	144.yyy	PM4 RIGHT side uplink (secret downlink)

PKT-DN	144.zzz	145.800	PM4 LEFT side downlink (secret uplink)

PKT-USA1	145.010	145.800

PKT-USA2	145.800	145.010

PKT-U1	43?.???	Same

PKT-U2	43?.???	Same

EXAMPLE OPERATING MODES:

The following pages include figures to illustrate some of the operating modes that will be used on ISS.