Modification List for TM-D700

REPEATER :

Article							Comment

In the Fixed Band Repeater mode, 'PTT'

is always allowed.

CTCSS tone when transmitting by 'PTT'.		If you enabled the TONE function

selected CTCSS tone is transmitted

when 'PTT'.

TRANSMISSION :

Article							Comment

TX frequency range is changed.

144-148 and 435-438.					You can not transmit if the current

frequency is out of this frequency

range in the VFO mode. But in the

Memory mode, you can enable per

channel by MCP software.(TX

permission)

APRS MODE :

Article							Comment

Message interruption display will not be cleared by 'PTT'.

All packets, also APRS messages, will be filtered by ALTNET .

Default Lat/Lon such as 0000.00/00000.00 will be transmitted in APRS mode.

Send out Satelite Element Data to the GPS jack, if the radio received an APRS

message which starts with ']' character.

PACKET MODE :

Change default parameters

Command		Parameter

BEACON		E 12

BTEXT		>ARISS - International Space Station (BBS/APRS on)

CHECK		7

CMSG			ON

CTEXT		ISS Crew Keyboard. Crew may not be available. For BBS/PMS

use RS0ISS-1

DAYUSA		OFF

ECHO			OFF

EPATH		SGATEE SGATE-10

FLOW			OFF

FULLDUP		ON

GPSTEXT		$GPGGA

HID			OFF

LPATH		APRS VIA SGATE,WIDE

LTEXT		Waiting for GPS Data

MAIL			ON

MALL			OFF

MBOD		ON

MYALIAS		NA1SS

MYCALL		RS0ISS RS0ISS-4

MYMCALL		RS0ISS-1 RS0ISS-11

NEWMODE		ON

NPATH		SGATEN SGATE-8

OVERKILL		10

PPERSIST		OFF

RETRY		5

SPATH		SGATES SGATE-9

TOUT			6 3

UIDIGI		ON ARISS,RELAY,WIDE,APRSAT

UIFLOOD		WIDE,NOID

UISSID		ON

UNPROTO		CQ VIA SGATE

WPATH		SGATEW SGATE-11

OTHERS :

Article					Comment

Default of Power On Message	Change to "ARISS" from "HELLO!!"

Hide frequency display		Always hide frequency, while the radio is in

Memory mode.

MCP

Article					Comment

TX permission		Added a "TX permission" check-box which permit

to transmit even if the frequency is out of the

transmission frequency range.

Packet mode				Added a "Packet Mode" check-box which has not

supported on the market version of MCP.

Beacon					Added a "BCON" check-box which has not

supported on the market version of MCP.

Panel Tab				Added this new tab. You can manage panel settings

such as current memory channel number, VFO

receiving band and TX power.

Tab name				Rename the tab names so that all tabs can be shown.

Tab names on the MCP main window was too long.

Therefore all tabs could not be shown

simultaneously.