Revised 27 AUG 90 - Minor type corrections, section 9 added.

Pacsat Protocol: File Transfer Level 0

Jeff Ward, G0/K8KA

Harold E. Price, NK6K

ABSTRACT

This document specifies Version 0 of the File Transfer Level 0

(FTL0) protocol designed for use on store-and-forward satellites

(PACSATs).  The protocol provides procedures for transferring

binary files to and from a server computer using an error-

corrected communication link.

Further to basic file transfer facilities, FTL0 provides file

selection and directory procedures.  Because a server may contain

many files, only some of which are of interest to each client, the

client may ``select'' a subset of the server's files.  Subset

selection is based on a flexible logical equation involving a

large number of file characteristics.  The scope of directory

requests or file download requests can be limited to those files

currently selected.

This protocol is designed to work specifically with files in which

the actual ``data'' or ``body'' of the file is preceded by a

standard ``header'', specified in the PACSAT File Header Defini-

tion document.

1.0 BACKGROUND

Terrestrial packet-radio bulletin board systems (PBBSs) in the amateur radio

service generally provide a character-based, human-to-machine interface

through which user's command the PBBS software.  A similar interface has been

adopted on the FUJI-OSCAR store-and-forward (PACSAT) satellites.  Through the

character-based interface, the user selects messages, requests directory list-

ings, and views, stores and deletes messages.  All messages are stored as

ASCII text files, and the PBBS maintains a directory entry for each message.

The directory contains message source, destination, title, time of storage,

size, and status.  The PBBS also stores data relating to each user station,

such as the most recent message each user station has listed or viewed.

Since messages and PBBS commands contain only ASCII characters, the user's

station equipment can be as basic as an ASCII terminal connected to a terminal

node controller (TNC). This simplicity makes the current system attractive,

and has lead to its widespread adoption.

These current PBBS systems also have several drawbacks, stemming from the

text-based user interface and from the lack of a flexible, informative message

header. All messages must be in ASCII, only simple ASCII message headers are

supported, and only a few search-keys are provided for message selection.

These limitations are particularly damaging in the PACSAT environment, where

the user base is large, the communications bandwidth is restricted, and the

onboard computer software must be compact and highly reliable.

The File Transfer Level 0 protocol defined here, and the PACSAT File Header

Definition presented elsewhere, combine to provide a PBBS protocol suite which

overcomes the limitations of the current text-based protocols.

The proposed protocols are based on a different model of a store-and-forward

messaging network. The PBBS is a ``server'' and the user stations are ``cli-

ents''. The server stores files, each of which contains a ``header'' of known

format, followed by a ``body''. The header holds information about the file

and about the body, including (but not limited to) the file length, creation

date, destination, source, keywords, and body compression technique. The body

is the data portion of the file, which can be any digital information, e.g.

binary, ASCII, FAX, digitised voice, etc.

The user station, or ``client'' is assumed to be under computer control.  The

client sends files to the server and receives files from the server over an

AX.25 data link, using an automated server/client file transfer protocol. The

client software interprets and displays the file headers and bodies for the

human user.  When the user wishes to upload a file to the server, client

software must add suitable standardized header fields to the file, and it may

also compress or otherwise prepare the file for uploading.

Each user is generally interested in only a small subset of the files avail-

able on a server.  Using a powerful selection facility the client can select

an interesting subset of the files on the server for directory listing or

downloading. The selection can be based on any combination of items in the

file header, thus user's software can assure that the user only ``sees'' files

which are likely to be of interest.

The server might not keep a user data base, and the client software must store

any important information concerning the user's last server session.  For

example, to implement a command equivalent to the PBBS command ``list new'',

the client software must store the message number, or date and time of the of

the last access to the server.

Since this protocol is also useful for terrestrial servers, we use the terms

``server'' and ``client'' throughout, instead of ``PACSAT'' and ``groundsta-

tion''.  The protocol does depend on the presence of the standard PACSAT file

header, this is to implement file locking or other features.  A version of the

protocol that can be used by terrestrial stations without the PACSAT headers

is under consideration.

FTL0 includes several features that make the protocol somewhat more complex

than some others in use in the amateur packet network.  These features include

simultaneous bidirectional file transfer, file locking by forwarding gateways,

and multiple destination forwarding.   As is sometimes the case, the actual

implementation is smaller than the specification.  The pseudo-code provided in

Appendix B and C show that the protocol is actually quite straightforward.

The authors also plan to make the prototype source code available on a public

BBS.

1.1 FTL0

The FTL0 protocol can support full-duplex message transfer, in which a single

data link (AX.25 connection) is used simultaneously to upload one file and to

download another file.  Client software does not necessarily have to implement

this capability.  The protocol provides for continuation of uploads and down-

loads interrupted by loss of the data link (generally caused by LOS at a

satellite groundstation), and includes confirmation handshakes at all critical

stages of file transfer.  An FTL0 file transfer should withstand interruption

at any stage.

It is assumed that the client software uses the transparent mode of an AX.25

TNC and that the AX.25 connection represents an error-free channel.  Other

data links which present an ordered, error corrected byte stream may be used.

Although the link may be terminated at any time, it is assumed that the link

will not pass corrupted or out-of-order bytes, and that processes at both ends

of the link will be notified if the link is terminated.  The PACSAT File

Header standard provides overall checks on the integrity of files transferred

to and from fully-compliant servers.

FTL0 commands and responses form simple ``packets'' inside the byte stream on

the communications link.  Specifically, in AX.25, an FTL0 packet may be trans-

mitted in one or more AX.25 I frames; neither the server nor the client is

aware of the AX.25 I frame boundaries.  The packet format is described in 2.0

below.

1.2 Associated Documents

PACSAT Data Specification Standards defines the meaning of data types and

structures within this specification.

The PACSAT File Header Definition defines the file header fields which must be

pre-pended to files before uploading to a fully-compliant FTL0 server.  (FTL0

procedures for terrestrial servers without PACSAT file headers are under

development.)

1.3 Terminology

server     is the PACSAT or the PBBS.

client     is the user station.

commands   are transmitted from the client to the server

responses  are transmitted from the server to the client

uplink     is data flow from the client to the server

downlink   is data flow from the server to the client

packet     is a sequence of bytes as defined in 2.0 below

frame     is an AX.25 level 2 I frame

data link  generally an AX.25 level 2 link, but may be a link using any

protocol which provides an ordered, error-checked, data transparent byte

stream.

1.4 Overview of Operation

To allow simultaneous uploading and downloading, FTL0 clients and servers

implement two state machines.  One is related to file uploads, and the second

to all other possible commands.  FLT0 is not symmetric; the protocol state

machines implemented by the server are necessarily different from those imple-

mented by the clients.

Dividing the data stream into packets allows for modular software implementa-

tion and the multiplexing of commands and data on a single data link.  The

packet format is simple.  Every packet begins with 2 bytes of control informa-

tion, which can be followed by 0 to 2047 bytes of data.  The packet may arrive

in many AX.25 I frames.  No packet synchronization or checksumming is provid-

ed, since the data link (AX.25 connection) provides a notionally error-free

connection.  Full file error checks are performed at the completion of file

transfers.

It is assumed that neither the client software nor the server software is

aware of or can control data link (AX.25) frame boundaries.

A FTL0 session starts when an data link is established between the server and

the client.  The server will send a LOGIN response when the connection is

established.

To allow the user to specify a group of files for directory listings or down-

loading, the client software implement may implement menus or some other user

interface.

Once the user has indicated the desired files, the client software converts

the user's preferences into an equation , such as

( (SOURCE = "G0K8KA*") && (KEYWORD = "*FTL0*") ) && (FILE_NUMBER > 3215).

And then encodes the equation in a postfix, binary format as a SELECT command.

When the server receives a SELECT command it builds a list of files for which

the equation evaluates to TRUE.  As a response to the SELECT command, the

server informs the client how many files are in the selection list.

When the client sends a one of the directory commands, the server responds by

transmitting directory information for the next 10 files on the selected list.

Each subsequent directory command results in the server transmitting directory

information for a further 10 files from the selection list, until the list is

exhausted.

Specific files or files from the selection list may be requested by the client

for downloading.  The download procedure consists of two command/response

cycles.  On the first cycle, the client requests the file and the server

responds by transmitting the file as a series of DATA packets followed by an

END_OF_DATA packet.  The server then waits for the client to acknowledge re-

ceipt of the file.  When the server receives the acknowledgement, it transmits

a final handshake and the download is finished.

To upload a file to the server, the client sends an UPLOAD command telling the

server how large the file is.  The server responds with a handshake which

either tells the client to abort or to proceed.  If instructed to proceed, the

client sends the entire file as a series of DATA packets followed by an

DATA_END packet.  The server receives the entire file, and then either ac-

knowledges the transfer or sends an error indication.

Because  PACSAT may go over the horizon in the midst of a  transaction,  down-

loading and uploading can be continued during several sessions with the serv-

er.  When a download is interrupted, the client must retain the file offset at

which communications was interrupted.  For interrupted uploads, the server

stores the appropriate offset.

1.5 File Identification

Although servers may identify files using some kind of system file name

(PACSATs use the MS-DOS standard of an 8 character body and a 3 character

extension), FTL0 does not use server file names to identify files.  Instead,

the server assigns every file a 32-bit file number.  This is to insure that

two files with different contents but the same name will never be confused.

Thus, every message which a user uploads to PACSAT (or another FTL0 server)

will have a unique identifier.

1.6 Gateway Procedures

Some messages on PACSAT will be destined for gateway stations which will

introduce the messages into other networks.  It is essential that only one

gateway download and relay each message.  To accommodate this, FTL0 provides

locked downloads.  Only one gateway at a time can perform a locked download of

a given file for a given destination.  If a locked download is interrupted,

this will be indicated in the PACSAT File Header of the file, so that other

gateways can tell when a file is in the process of being delivered.

PACSAT File Headers allow files with multiple destinations, and separate locks

are implemented for each destination.  See Appendix A for further information.

1.7 Delivery Registration

A simple form of delivery registration is supported by FTL0.  In the final

handshake after receiving a file, the client can command the server to modify

the contents of the PACSAT File Header to show the AX.25 address of the client

and the time at which the download was completed.  It is intended that this

facility be used if the file is specifically addressed to the client station,

not for files of general interest.  If the file has multiple destinations, one

``registration'' is supported for each destination.  See Appendix A for fur-

ther information.

1.8 State Variables

In order to support full-duplex operations, the server and the client maintain

2 state variables, one relating to the uploading of files and the other relat-

ing to all other operations (selecting, directories, and downloading).  These

are called the uplink state and the downlink state variables.

Appendix B provides pseudo-code definitions of the state transitions for

servers, Appendix C is for clients.

2.0 PACKET FORMAT

An FTL0 packet is a sequence of information bytes preceded by a two byte

header.  After establishment of a data link, the first byte delivered to

either client or server is the first byte of a packet header.  The packet

header informs the client/server the type of the packet and the number of

information bytes which will follow.  There may be between 0 and 2047 informa-

tion bytes, inclusive.  After reception of the final information byte of a

packet, the client/server expects the next byte to be the first byte of anoth-

er packet header.  Such a stream is shown below, where [<info>] indicates that

there may be no data bytes.

<length_lsb><h1>[<info>...]<length_lsb><h1>[<info>...]

|----First FTL0 packet-----|----Second FTL0 packet---|

2.1 Header Format

struct FTL0_PKT {

unsigned char length_lsb;

unsigned char h1;

}

<length_lsb>  - 8 bit unsigned integer supplying the least significant 8  bits

of data_length.

<h1> - an 8-bit field.

bits 7-5 contribute 3 most significant bits to data_length.

bits 4-0 encode 32 packet types as follows:

0    DATA

1    DATA_END

2    LOGIN_RESP

3    UPLOAD_CMD

4    UL_GO_RESP

5    UL_ERROR_RESP

6    UL_ACK_RESP

7    UL_NAK_RESP

8    DOWNLOAD_CMD

9    DL_ERROR_RESP

10   DL_ABORTED_RESP

11   DL_COMPLETED_RESP

12   DL_ACK_CMD

13   DL_NAK_CMD

14   DIR_SHORT_CMD

15   DIR_LONG_CMD

16   SELECT_CMD

17   SELECT_RESP

All other values reserved

2.2 Information Length

The <information_length> value is formed by pre-pending bits 7-5 of the <h1>

byte to the <length_lsb> byte.  <information_length> indicates how many more

bytes will be received before the beginning of the next packet.  If <informa-

tion_length> is 0, there are no information bytes.

3.0 LOGON

As soon as a data link is established between the client and the server, the

client is considered to be logged on.  The server sends a LOGIN_RESP packet

with one byte of flags and a 4-byte timestamp.

When the client receives the LOGIN_RESP packet the server is initialized and

ready to begin transactions.

3.1 Initiation of Session

An FTL0 session is initiated when a data link is established between the

client and the server.

3.2 Server Login Response Packet

When the data link is established the server transmits a LOGIN_RESP packet.

Packet: LOGIN_RESP

Information: 5 bytes

struct LOGIN_DATA{

unsigned long login_time;

unsigned char login_flags;

}

<login_time>  -  a 32-bit unsigned integer indicating the  number  of  seconds

since January 1, 1970.

<login_flags> - an 8-bit field.

bit:76543210

xxxxSHVV

Bit 3, the SelectionActive bit, will be 1 if there is already an active selec-

tion list for this client.  The SelectionActive bit will be 0 if there is no

active selection for this client already available.

Bit 2, the HeaderPFH bit, will be 1 if the server uses and requires PACSAT

File Headers on all files.  If the HeaderPFH bit is 1, the flag PFHserver used

in the following definition should be considered TRUE.

The HeaderPFH bit will be 0 if the server does not use PACSAT File Headers.

If the HeaderPFH bit is 0, the modified procedures specified in Section 7

should be used.

Bits 1 and 0 form a 2-bit FTL0 protocol version number.  The version described

here is 0.

3.3 Server Login Initialization

Upon transmitting the LOGIN_RESP packet the server should initialize its state

variables to UL_CMD_WAIT and DL_CMD_WAIT.

3.4 Client Login Initialization

Upon receiving the LOGIN_RESP packet, the client should initialize its state

variables to UL_CMD_OK and DL_CMD_OK.

4.0 SELECT

The select command is the mechanism through which the client can search the

server for desirable files.  The human user can search the files on the server

based on any information contained in the PACSAT File Header.  To achieve this

goal, the implementation and specification go through several levels of ab-

straction which must be followed closely.

The user's desires form an equation.  The equation might simply be ``the file

is to G0K8KA and it was stored after my last logon''.  Or it might be more

complex, e.g. ``the file is less than 8 kbytes, and it was created after 9

July 1990, and it has ``landrover'' as one of its keywords''.  Humans would

naturally express this in ``infix'' notation:

(FILE_SIZE < 8k) && (CREATE_DATE > 9 July 1990) && (KEYWORDS == "*landrover*")

The server uses ``postfix'' notation, like an RPN calculator, and wants an

equation like:

(FILE_SIZE < 8k) (CREATE_DATE > 9 july 1990) && (KEYWORDS == "*landrover*") &&

To convert a postfix logical equation into a SELECT command, the client must

encode the logical operators, the relational operators the header item names,

and the values to compare against.  The header item names and comparison

values are encoded just as they are in the PACSAT File Header definition.  The

logical operators and relational operators are encoded as single bytes.  The

complete syntax is defined below.

4.1 Client Issuing Select Command

When the client's downlink state variable is DL_CMD_OK (e.g. the client is not

involved in another select, file download, or directory download), the client

may transmit a SELECT_CMD packet.

Packet: SELECT_CMD

Information: variable length

SELECTION

SELECTION is defined recursively by the following structures and rules.

struct SELECTION {

struct LVALUEx equation;

unsigned char end_flag;

}

<end_flag> - 0x00

<equation> is recursively defined by

two structures LVALUE0 and LVALUE1:

struct LVALUE0 {

struct LVALUEx t1;

struct LVALUEx t2;

unsigned char lop;

}

<t1> and <t2> are further LVALUE0 or LVALUE1 structures.

<lop> is and 8-bit field representing a logical operator:

bit 76543210

00000001     is AND

00000010     is OR

struct LVALUE1 {

unsigned char relop;

unsigned int item_id;

unsigned char length;

unsigned char constant[];

}

<relop> is an 8-bit field encoding a relational operator:

bit7

0

bit654  Relational Operator

000  equal to

001  greater than

010  less than

011  not equal

100  greater than or equal to

101  less than or equal to

110  reserved

111  reserved

bit3210 Type of Comparison

0000 treat <constant> as a

multi-byte unsigned

integer (valid for

<length> 1,2, and 4).

0001 treat <constant> as a

multi-byte signed

integer (valid for

<length> 1,2,and 4).

0010 treat <constant> as an

array                of

unsigned char

0011 treat <constant> as an

array of ASCII

characters, convert to

lower case before making

comparison.

0100 treat <constant> as an

array of ASCII

characters, convert to

lower case before

comparison and interpret

wildcard characters.

ALL OTHER VALUES RESERVED.

<item_id> is a 16-bit unsigned integer identifying one of the PACSAT Header

Definition header items.

<length> is the number of bytes in the <constant[ ]> array.

<constant[ ]> is an array of bytes which are compared against the header item

identified by <item_id>, using the specified relational operator and compari-

son type.

4.2 Response to Select Command

The server responds with one of the packets from 4.2.1 or 4.2.2 and returns

its downlink state to DL_CMD_OK, ready to accept another command from the

client.

4.2.1 Successful Selection

If the server can interpret the SELECT_CMD packet, it responds with a

SELECT_RESP packet

Packet: SELECT_RESP

Information: 2 bytes

unsigned int no_of_files

<no_of_files> is a 16 bit unsigned integer telling how many files have been

selected.  It may be 0.

4.2.2 Unsuccessful Selection

If the server cannot interpret the SELECT_CMD packet, it responds with a

DL_ERROR_RESP packet.

Packet: DL_ERROR_RESP

Information: 1 byte

unsigned char err_code

<err_code> is ER_POORLY_FORMED_SEL if the selection equation could not be

parsed by the server because of a syntax error.

5.0 DOWNLOAD

To receive a file from the server, the client uses the download command.  This

command can be used to download a specific file, to continue the downloading

of a specific file, or to download the next file in the selection list.

5.1 Client Initiates Download

When the client's uplink state variable is DL_CMD_OK, the client may send the

DOWNLOAD_CMD packet.

Packet: DOWNLOAD_CMD

Information : 9 bytes of arguments with the following structure

struct {

unsigned long file_no;

unsigned long byte_offset;

unsigned char lock_destination;

}

<file_no> is a 32-bit binary integer uniquely identifying a file on the serv-

er.  Two reserved values have special meanings: If <file_no> is equal to

0xffffffff, the server will attempt to use the next file in the current selec-

tion list, moving from older files to newer files.  If <file_no> is equal to

0, the server will attempt to use the next file in the current selection list,

moving from newer files toward older files.  If <file_no> is not one of the

reserved values, the server attempts to download the file which it associates

with <file_no>.

<byte_offset> is a 32-bit unsigned binary integer giving the offset from the

beginning of the file at which the download should begin.  If the client is

continuing an aborted download, this argument should be set to the number of

bytes previously received.  Otherwise it should be 0.

<lock_destination> is an 8 bit unsigned binary integer.  It will generally be

set to 0 by non-gateway stations.

[If <lock_destination> is not 0, it indicates that the client wishes to con-

duct a locked download for the destination numbered by <lock_destination>.  If

the <lock_destination> exists and is not already locked, the client's AX.25

address is placed in the PACSAT File Header AX25_DOWNLOADER item associated

with the specified DESTINATION item.  Destination numbering is defined in

section Appendix A.]

5.2 Server Responses to the Download Command

When it receives a DOWNLOAD_CMD packet the server determines whether the

download is possible.  This may include determination of the desired file

number from the current select list.

5.2.1  Downlinking File Data

If the download is possible the server transmits the file data beginning

<byte_offset> bytes from the beginning of the file.  If <byte_offset> is

greater than or equal to the file length, no data is transmitted.  The file

data bytes are transmitted as information in DATA packets.  These packets may

be any size from 0 to 2047 bytes.

[If <lock_destination> is not-zero, the server sets the <DOWNLOAD_TIME> asso-

ciated <lock_destination> before beginning to send DATA packets.]

Once the server has transmitted a DATA packet containing the last byte of the

file, or if the <byte_offset> was greater than or equal to the file length,

the server transmits a single DATA_END packet.  After transmitting the end

packet, the server expects to receive a DL_ACK_CMD or DL_NAK_CMD.

5.2.2 Failure of Download Command

If the server cannot service the download command, it responds with a

DL_ERROR_RESP packet.

Packet: DL_ERROR_RESP

Information: 1 byte

unsigned char err_code;

<err_code> is an 8-bit unsigned binary integer will be one of:

ER_SELECTION_EMPTY if the selection list has no more files in it or no select

command was previously received.

ER_NO_SUCH_FILE_NUMBER if the file identified by <file_no> does not exist.

[ER_ALREADY_LOCKED - <lock_destination> was not 0, and the file is already

locked for the specified destination.

ER_NO_SUCH_DESTINATION - <lock_destination> was not 0, and the file has fewer

than <lock_destination> DESTINATION items in its PACSAT File Header.]

5.3 Client Accepting or Rejecting Downloaded File

5.3.1 Successful Download

Upon reception of the DATA_END packet from the server, the client performs any

possible checks on the received file.  If the file passes the checks, the

client transmits a DL_ACK_CMD packet.

Packet: DL_ACK_CMD

Information: 1 byte

unsigned char <register_destination>

<register_destination> is an 8-bit unsigned integer identifying one of the

destinations in the PACSAT File Header of the downloaded file.

<register_destination> should be 0 if the client does not wish to register

receipt of the file.

Upon receiving the DL_ACK_CMD packet, the server completes the download proc-

ess.

[If <register_destination> is not 0, the appropriate DESTINATION item in the

PACSAT File Header is located and the associated AX25_DOWNLOADER and

DOWNLOAD_TIME items are updated.

If <lock_destination> was non-zero in the DOWNLOAD_COMMAND, the server locates

the appropriate DESTINATION item in the PACSAT File Header and updates the

associated DOWNLOAD_TIME item.]

5.3.2 Unsuccessful or Aborted Download

If the client wishes to abort the download before receiving the DATA_END

packet from the server, or finds that the downloaded file fails some integrity

test after reception of the DATA_END packet, the client sends the DL_NAK_CMD

packet.

Packet: DL_NAK_CMD

Information: none

Upon receiving this packet, if the server has not already sent the DATA_END

packet, it will do so and proceed to the final unsuccessful download handshake

(5.4.2).  If DL_NAK_CMD is received after the server has transmitted the

DATA_END packet, the server proceeds to the final download handshake.

5.4 Final Download Handshake

5.4.1 Completion of Successful Download

[If the server receives the DL_ACK_CMD, it removes any <lock_destination> lock

from the file and sets <AX25_DOWNLOADER> and <DOWNLOAD_TIME> for the specified

<DESTINATION> item.]

[If <register_destination> in the DL_ACK_CMD is non-zero, the server attempts

to set the <AX25_DOWNLOADER> and <DOWNLOAD_TIME> items associated with the

specified <DESTINATION> item.  If the <register_destination> does not exist,

the server transmits a DL_ABORTED_RESP packet.]

If <register_destination> was 0, the server transmits a DL_COMPLETED_RESP

packet, and returns its downlink state variable to DL_CMD_OK.

Packet: DL_COMPLETED_RESP

Information: none.

This response tells the client that the server has completed processing the

DL_ACK_CMD.  The client downlink state variable should be set to DL_CMD_OK.

If the L2 link fails before the client receives the DL_DONE_RESP, the client

cannot be sure the DL_ACK_CMD was processed.  This is important if the client

had specified a <lock_destination> or a <register_destination>.  In either of

these cases, the client should treat the download as incomplete and continue

it later.  This assures that the PACSAT File Header has been properly modi-

fied.  If both <lock_destination> and <register_destination> were 0, the

client can consider the download complete.

5.4.2 Completion of Unsuccessful Download

A download is ``unsuccessful'' if the server receives the DL_NAK_CMD from the

client or if <register_destination> is non-zero and indicates a non-existent

destination.

If <lock_destination> was non-zero, the lock is removed and <DOWNLOAD_TIME> is

updated.

The server transmits the DL_ABORTED_RESP packet.

Packet: DL_ABORTED_RESP

Information: none

The server downlink state variable returns to the DL_CMD_OK state.

Upon reception of the DL_ABORTED_RESP the client downlink state variable

returns to DL_CMD_OK.

6.0 DIRECTORY

The client uses directory commands to get information about files on the

server.  Servers conforming to the PACSAT File Header Definition send PACSAT

File Headers as directory entries.  ``Long'' directory entries are the com-

plete PACSAT File Header and ``short'' directory entries are only the Mandato-

ry File Header items.

The client can request a directory entry for a specific file or for the files

in the selection list.  The selection list can be scanned from oldest to

newest files or from newest to oldest files.

6.1 Initiating a Directory

The client may request a directory whenever the client downlink state variable

is DL_CMD_OK.  To request a directory, the client transmits either a

DIR_LONG_CMD packet or a DIR_SHORT_CMD packet.

Packet: DIR_LONG_CMD

or DIR_SHORT_CMD

Information: 4 bytes

unsigned long file_no;

<file_no> is a 32-bit unsigned binary integer identifying a file or files on

the server.  There are two reserved values: 0 and 0xffffffff.  If <file_no> is

0xffffffff, the server will send directory entries for the next 10 files in

the current selection list, moving from older files to newer files.  If

<file_no> is 0, the server will send directory entries for the next 10 files

in the current selection list, moving from newer files toward older files.

6.2  Server Responses to Directory Commands

6.2.1 Successful Directory Request

If the directory request can be serviced (there are files in the selection

list, or a specified file exists) the server will transmit one or more PACSAT

File Headers in a series of DATA packets followed by a DATA_END packet.  A

maximum of 10 file headers will be transmitted for each DIR command.

6.2.2. Failed Directory Request

If the directory request cannot be serviced, a DL_ERROR_RESP packet is trans-

mitted.

Packet: DL_ERROR_RESP

Information: 1 byte

unsigned char err_code;

<err_code> will be one of:

ER_SELECTION_EMPTY - if the <file_no> was 0 or 0xffffffff, and either there

are no files left in the selection list or there is no selection list.

ER_NO_SUCH_FILE_NUMBER - if <file_no> is not 0 or 0xffffffff and no file

identified by <file_no> exists on the server.

7. UPLOAD

7.1. Initiating an Upload

The client can initiate an upload any time the client's upload state variable

is UL_CMD_OK.

Packet: UPLOAD_CMD

Information: 8 bytes

struct {

unsigned long continue_file_no;

unsigned long file_length;

}

<continue_file_no> - a 32-bit unsigned integer identifying the file to contin-

ue.   Used to continue a previously-aborted upload.  Must be 0 when commencing

a new upload.

<file_length> -  32-bit unsigned integer indicating the number of bytes in the

file.

7.2 Server Responses to Upload requests

Downlink Packet: UL_GO_RESP or UL_ERROR_RESP

7.2.1 Successful Upload Request

Packet: UL_GO_RESP

Information: 8 bytes

struct {

unsigned long server_file_no;

unsigned long byte_offset;

}

<server_file_no> is a 32-bit unsigned binary integer identifying the client's

file on the server.

<byte_offset> is a 32-bit unsigned binary integer number of bytes from the

beginning of the file at which the client should begin file transmission. This

will be 0 if the <continue_file_no> was zero.

After receiving the UPLOAD_PROCEED_RESP packet, the client should advance to

the data transmitting state described in Section 7.3.

7.2.2 Unsuccessful Upload Request

If the server cannot process the upload request, it will transmit an

UL_ERROR_RESP packet.

Packet: UL_ERROR_RESP

Information: 1 byte

unsigned char err_code;

<err_code> must be one of:

ER_NO_SUCH_FILE_NUMBER if <continue_file_no> is not 0 and the file identified

by <continue_file_no> does not exist.  Continue is not possible.

ER_BAD_CONTINUE if <continue_file_no> is not 0 and the <file_length> does not

agree with the <file_length> previously associated with the file identified by

<continue_file_no>.  Continue is not possible.

ER_FILE_COMPLETE if <continue_file_no> is not 0 and the file identified by

<continue_file_no> was completely received on a previous upload.  Note -

receipt of this command should be accepted by the client as confirmation of

file receipt by the server.

ER_NO_ROOM if the server does not have room for the file.

After transmitting the UL_ERROR_RESP packet, the server's uplink state varia-

ble is set to UL_CMD_OK.

7.2.3  Continuation of Uploads

Any file for which the client receives a UL_GO_RESP should be continued until

a UL_ACK_RESP or a non-recoverable UL_ERR_RESP or UL_NAK_RESP for that file is

received.  The server should be prepared to continue reception of any file for

which it has transmitted a UL_GO_RESP.  Some file numbers will be allocated by

the server and lost by link failure before the UL_GO_RESP reaches the client;

a 32-bit file number space provides sufficient scope for some lost file num-

bers.  To avoid saving partial files for these lost file numbers, the server

should not reserve space for a message until it has received at least one DATA

packet from the client.

7.3. Data Uplinking Stage

The client now uplinks the bytes from the file, in DATA packets.  The uplink-

ing should begin <byte_offset> bytes from the start of the file.  After trans-

mitting a DATA packet containing the last byte in the file, the client should

transmit a DATA_END packet.  The client should also transmit a DATA_END packet

if the <byte_offset> was greater than or equal to the file length.

The server may attempt to terminate the upload by transmitting a UL_NAK_RESP

at any time during the upload.  If the client receives a UL_NAK_RESP packet

before transmitting a DATA_END packet the client must send a DATA_END packet.

7.4. Completion of Upload

7.4.1 Successful Upload Completion

When the server receives the DATA_END packet it will check the integrity of

the file as far as possible.  If the checks pass, the server will downlink a

UL_ACK_RESP packet.

Packet: UL_ACK_RESP

Information: none

After transmitting the UL_ACK_RESP the server uplink state variable is

UL_CMD_OK.  After receiving the UL_ACK_RESP, the client uplink state variable

is UL_CMD_OK.

7.4.2 Failure Caused by Server Rejecting Upload

The server may reject an upload while the client is sending DATA packets (due

to file system problems on the server) or after the client has sent the

DATA_END packet (due to corruption of the file).

If the server must abort the upload while receiving DATA packets or after

receiving the DATA_END checks fail, it sends the UL_NAK_RESP packet.

Information: 1 byte

unsigned char err_code;

<err_code>     must be one of:

ER_BAD_HEADER - The file either has no PFH, or has a badly-formed PFH.

ER_HEADER_CHECK - The PFH checksum failed.

ER_BODY_CHECK - The PFH body checksum failed.

ER_NO_ROOM - The server ran out of room for file storage before the upload was

complete.  The server will implement procedures to avoid frequently running

out of room, but this cannot be guaranteed.

After transmitting the UL_NAK_RESP packet, the server uplink state variable is

UL_CMD_OK.  After receiving the UL_ERROR_RESP, the client uplink state varia-

ble is UL_CMD_OK.

7.4.3 Link Failure During Upload

If the data link fails before the server receives the DATA_END packet, the

server retains the offset of the next byte needed for the file.  This value

will be transmitted by the server as <byte_offset> if the client later contin-

ues the upload by specifying the <server_file_no> in the <continue_file_no> of

an UPLOAD_CMD packet.  If the offset is not 0, the server may safely retain a

temporary file containing the data received so far.  If the server retains

files for which the <byte_offset> is 0, these should be purged after some

reasonable time, since the client may not know the <server_file_no> of the

file.

8.0 TERMINATION OF SESSION

The session is terminated by closing the data link.  Either the client or the

server may decide to close the link when uplink state is UL_CMD_OK and down-

link state is DL_CMD_OK.  The link is also terminated if unexpected packets

are received.

If the link terminates when any state machine is not in the _CMD_OK state,

appropriate actions are taken.

9.0 OTHER ERROR CONDITIONS

There are other error conditions which might occur when a server is processing

commands.  Such errors are indicated by DL_ERROR_RESP or UL_ERROR_RESP packets

with appropriate error codes.  Robust client implementations will be prepared

to handle such packets at any time during the session.

Specific codes which have currently been assigned are ER_ILL_FORMED_CMD and

ER_SERVER_FSYS.

9.1 ER_ILL_FORMED_CMD

This error code is transmitted by the client when a command is received with

too many or too few data bytes, and when an unrecognized command is received.

The current command is aborted and the appropriate server state machine is

returned to the _CMD_OK state.

9.2 ER_SERVER_FSYS

This indicates a failure in the server file system, e.g. no file handles

available for the server to complete the requested operation.  The current

command is aborted, and the appropriate server state machine is returned to

the _CMD_OK state."

APPENDIX A - Use of the PACSAT File Header

The PACSAT File Header Definition defines a standard header which will be

found at the beginning of every file downloaded from PACSAT (or any fully

compliant FTL0 server).

One purpose of the PACSAT File Header is to allow clients to determine the

status of files on the server.  Specifically, the originating client may wish

to find out if the file has been downloaded by its intended destination, and a

Gateway client may wish to find out if a message still needs to be downloaded

for relay to a particular destination.  Both of these tasks are done by exam-

ining a set of PACSAT File Header Items comprising <DESTINATION>, <AX25_DOWN-

LOADER> and <DOWNLOAD_TIME>.

A gateway will examine <DESTINATION> to see if it can relay or deliver the

file.  If so, it will examine <DOWNLOAD_TIME> and <AX25_DOWNLOADER> to deter-

mine the delivery status of the message:

If <DOWNLOAD_TIME> is zero, the message still needs to be relayed.

If <DOWNLOAD_TIME> is non-zero, but <AX25_DOWNLOADER> is blanks, the message

is in the process of being downloaded, and the download started at

<DOWNLOAD_TIME>.

If <DOWNLOAD_TIME> is non-zero, and <AX25_DOWNLOADER> is non-blank, the mes-

sage has been downloaded for relay to the <DESTINATION>.

A similar (but simpler) procedure is used to see if a specific destination

station has ``registered receipt'' of a file.  When receipt is registered (at

the end of a successful download), <AX25_DOWNLOADER> and <DOWNLOAD_TIME> will

both be correctly set.

For these checks to work correctly, the downloading clients must use the

proper command procedures.  Gateways must set the correct <lock_destination>

in their DOWNLOAD_CMD packets, and clients must set the correct

<register_destination> in the DL_ACK_CMD.  The <lock_destination> procedure

should be implemented by all gateways. The <register_destination> procedure is

optional for client implementations.

The proper number for <register_destination> or <lock_destination> is deter-

mined as follows.  If a file has only one <DESTINATION> item, it is destina-

tion 1.  Any other destinations are numbered sequentially in order of occur-

rence in the PACSAT File Header.

APPENDIX B - Server State Transition Definitions

SERVER STATE DEFINITIONS

The following pseudo code defines the transitions of the Server state machines

in FTL0.  One state machine is used for file uploading.  The other state

machine is used for all other procedures.

It is assumed that the data link will be terminated if there have been no

packets transmitted or received for a period of time (TBD).  If the link is

terminated by this ``activity timeout'' both state machines receive a ``Data

Link Terminated'' event.  ``Data Link Terminated'' is also generated if the

link fails for any reason.

When an unexpected packet is received by one of the state machines, that state

machine terminates the data link and returns to an uninitialized state.  The

second state machine is notified of this through a ``Data Link Terminated''

event, and also returns to an uninitialized state.  At this point, the FTL0

processing for this client ceases until the establishment of another data

link.

STARTUP

State variables are created when a data link is established between client and

server.  The server executes the following:

Transmit LOGIN_RESP packet.

ul_state := UL_CMD_OK

dl_state := DL_CMD_OK

SERVER UPLINK STATE MACHINE

This machine receives only frames which are relevant to the file uploading

process : UPLOAD_CMD, DATA, and DATA_END.  It also is executed upon timeout of

the input timer or termination of the level 2 link (from local or remote

causes).

state UL_CMD_OK {

EVENT: Receive UPLOAD_CMD packet{

if ok to upload

Send UPLOAD_GO_RESP packet.

ul_state <- UL_DATA_RX

else

Send UL_ERROR_RESP packet.

ul_state <- UL_CMD_OK

}

EVENT: DEFAULT{

if EVENT is not "data link terminated"

Terminate data link.

ul_state <- UL_UNINIT

}

}

state UL_DATA_RX {

EVENT: Receive DATA packet {

Try to store data.

if out of storage {

Transmit UL_NAK_RESP packet.

Close file.

Save file_offset and file_number.

ul_state <- UL_ABORT

}

else {

Update file_offset.

ul_state <- UL_DATA_RX

}

}

EVENT: Receive DATA_END packet {

Close file.

if file passes checks {

Transmit UL_ACK_RESP packet

ul_state <- UL_CMD_OK

}

else {

file_offset <- 0

Save file_offset and file_number

Transmit UL_NAK_RESP packet

ul_state <- UL_CMD_OK

}

}

EVENT: DEFAULT{

if (file_offset > 0)

Save file_offset and file number.

if EVENT is not "data link terminated"

Terminate data link.

ul_state <- UL_UNINIT

}

}

state UL_ABORT {

EVENT: Receive DATA packet

ul_state <- UL_ABORT

EVENT: Receive DATA_END packet

ul_state <- UL_CMD_OK

EVENT: DEFAULT{

if EVENT is not "data link terminated"

Terminate data link.

ul_state <- UL_UNINIT

}

}

SERVER DOWNLINK STATE MACHINE

This machine receives all packets other than UPLOAD_CMD, DATA, and DATA_END.

state DL_CMD_OK {

EVENT: Receive SELECT_CMD packet {

if syntax is correct {

Form select list.

Transmit SELECT_RESP packet.

dl_state <- DL_CMD_OK

}

else {

Transmit DL_ERROR_RESP packet.

dl_state <- DL_CMD_OK

}

}

EVENT: Receive DIR_LONG_CMD or DIR_SHORT_CMD packet {

if there are directories to send

dl_state <- DL_DIR_DATA

else {

Transmit DL_ERROR_RESP packet.

dl_state <- DL_CMD_OK

}

}

EVENT: Receive DOWNLOAD_CMD packet {

if request can be serviced  {

seek to <offset>

if <lock_destination> <> 0

set PFH DOWNLOAD_TIME.

dl_state <- DL_FILE_DATA

}

else {

Transmit DL_ERROR_RESP packet.

dl_state <- DL_CMD_OK

}

}

EVENT: Receive DL_NAK_CMD packet

dl_state <- DL_CMD_OK

EVENT: DEFAULT{

if EVENT is not "data link terminated"

Terminate data link.

dl_state <- DL_UNINIT

}

}

state DL_DIR_DATA {

if there is more directory data to send {

Transmit directory data in DATA packets.

dl_state <- DL_DIR_DATA

}

else {

Transmit DATA_END packet.

dl_state <- DL_CMD_OK

}

EVENT: Receive DL_NAK_CMD packet{

Transmit DATA_END packet.

dl_state <- DL_CMD_OK.

}

EVENT: DEFAULT{

if EVENT is not "data link terminated"

Terminate data link.

dl_state <- DL_UNINIT

}

}

state DL_FILE_DATA {

if there is more file data to send

Transmit file data in DATA packets.

dl_state <- DL_FILE_DATA

}

else {

Transmit DATA_END packet.

dl_state <- DL_FILE_END

}

EVENT: Receive DL_NAK_CMD packet {

If lock_destination <> 0

unlock lock_destination.

Transmit DATA_END packet.

Transmit DL_ABORTED_RESP packet.

dl_state <- DL_CMD_OK

}

EVENT: DEFAULT {

If lock_destination <> 0

unlock lock_destination.

if EVENT is not "data link terminated"

Terminate data link.

dl_state <- DL_UNINIT

}

}

state DL_FILE_END {

EVENT: Receive DL_NAK_CMD packet {

dl_state <- DL_CMD_OK

Transmit DL_ABORTED_RESP packet.

If lock_destination <> 0

unlock lock_destination.

}

EVENT: Receive DL_ACK_CMD packet {

if register_destination <> 0 and register_destination exists {

set PFH AX25_DOWNLOADER

set PFH DOWNLOAD_TIME

Transmit DL_COMPLETED_RESP packet.

}

else if register_destination <> 0 and it doesn't exist {

Transmit DL_ABORTED_RESP.

}

else if lock_destination <> 0 {

unlock lock_destination.

set PFH AX25_DOWNLOADER.

set PFH DOWNLOAD_TIME.

Transmit DL_COMPLETED_RESP packet.

}

dl_state <- DL_CMD_OK

}

EVENT: DEFAULT {

If lock_destination <> 0

unlock lock_destination.

if EVENT is not "data link terminated"

Terminate data link.

dl_state <- DL_UNINIT

}

APPENDIX C - Client State Transition Definitions

Client Pseudo-code State diagrammes.

The following diagrammes indicate how an FTL0 client should react to different

events when in different states.

``User Requests'' come from the process using the FTL0 state machine.

Receive packets come from the data link and transmit packets go to the data

link.

It is assumed that the data link will be terminated if there have been no

packets transmitted or received for a period of time (TBD).  If the link is

terminated by this ``activity timeout'' the state machines receive a ``Data

Link Terminated'' event.  ``Data Link Terminated'' is also generated if the

link fails for any reason.

UPLINK STATE MACHINE

The client uplink state machine controls the file uploading process.  File

downloading and all other processes are controlled by the downlink state

machine.

The state of the uplink is indicated by the ul_state variable.

The uplink state machine processes the following events:

EVENT: User Requests Uplink

EVENT: Data Link Terminated

EVENT: Receive UL_GO_RESP

EVENT: Receive UL_ERROR_RESP

EVENT: Receive UL_ACK_RESP

EVENT: Receive UL_NAK_RESP

Where the ``EVENT: DEFAULT'' is indicated, this includes all events on the

above list which have not already been processed.

State UL_UNINIT {

EVENT: User Requests File Upload

Refuse.

EVENT: DEFAULT

ignore.

}

State UL_CMD_OK {

EVENT: User Requests File Upload {

Transmit UL_CMD packet, setting <continue_file_no> if necessary.

ul_state <- UL_WAIT

}

EVENT: DEFAULT{

if EVENT is not "data link terminated"

Terminate data link.

ul_state <- UL_UNINIT

}

}

State UL_WAIT {

EVENT: Receive UL_GO_RESP packet. {

Associate <server_file_number> with the file.

Mark the file CONTINUE.

ul_state <- UL_DATA

}

EVENT: Receive UL_ERROR_RESP packet. {

If error is unrecoverable, mark the file IMPOSSIBLE.

ul_state <- UL_CMD_OK

}

EVENT: DEFAULT{

if EVENT is not "data link terminated"

Terminate data link.

ul_state <- UL_UNINIT

}

}

State UL_DATA {

If there is data to send {

Transmit DATA packet.

ul_state <- UL_DATA.

}

else {

Transmit DATA_END packet.

ul_state <- UL_END.

}

EVENT: Receive UL_NAK_RESP packet. {

if error is unrecoverable mark file IMPOSSIBLE.

Transmit DATA_END packet.

ul_state <- UL_CMD_OK.

}

EVENT: DEFAULT{

if EVENT is not "data link terminated"

Terminate data link.

ul_state <- UL_UNINIT

}

}

State UL_END {

EVENT: Receive UL_ACK_RESP packet. {

Mark file COMPLETED.

ul_state <- UL_CMD_OK

}

EVENT: Receive UL_NAK_RESP packet. {

If error is unrecoverable mark file IMPOSSIBLE.

ul_state <- UL_CMD_OK

}

}

DOWNLINK STATE MACHINE

The client's downlink state machine processes the following events:

EVENT: User requests download

EVENT: User requests directory

EVENT: User requests selection

EVENT: User requests abort

EVENT: Receive DL_ABORTED_RESP packet.

EVENT: Receive DL_COMPLETED_RESP packet.

EVENT: Receive SELECT_RESP packet.

EVENT: Receive LOGIN_RESP packet.

EVENT: Receive DATA packet.

EVENT: Receive DATA_END packet.

EVENT: Receive DL_ERROR_RESP packet.

The only user request which generates and event when not in DL_CMD_OK state is

``user requests abort'', which is processed in the DL_DATA state.  All other

requests are ignored (perhaps queued) until dl_state is DL_CMD_OK.

State DL_UNINIT{

}

State DL_CMD_OK{

EVENT: User requests download {

Transmit DL_CMD packet.

dl_state := DL_WAIT.

}

EVENT: User requests directory {

Transmit DIR_LONG_CMD or DIR_SHORT_CMD packet.

dl_state := DL_DIR_WAIT.

}

EVENT: User requests selection {

Transmit SELECT_CMD packet.

dl_state := DL_SEL.

}

EVENT: User requests abort {

ignore.

dl_state := DL_CMD_OK

}

EVENT: DEFAULT{

if EVENT is not "data link terminated"

Terminate data link.

dl_state <- UL_UNINIT

}

}

State DL_WAIT{

EVENT: Receive DL_ERROR_RESP packet {

dl_state <- DL_CMD_OK.

}

EVENT: Receive DATA packet. {

Store data.

Mark file INCOMPLETE.

dl_state <- DL_DATA.

}

EVENT: Receive DATA_END packet. {

If file at client is ok {

Mark file DATA_OK.

Save <continue_offset> equal to file length.

Transmit DL_ACK_CMD.

}

Else {

Mark file INCOMPLETE.

Save <continue_offset> of 0.

Transmit DL_NAK_CMD.

}

dl_state <- DL_END.

}

EVENT: DEFAULT{

if EVENT is not "data link terminated"

Terminate data link.

dl_state <- UL_UNINIT

}

}

State DL_DATA{

EVENT: User requests abort {

Save current offset as <continue_offset>.

Transmit DL_NAK_CMD.

dl_state <- DL_ABORT.

}

EVENT: Receive DATA packet. {

Store data.

dl_state <- DL_DATA.

}

EVENT: Receive DATA_END packet. {

If file is ok at client

Transmit DL_ACK_CMD packet.

Else

Transmit DL_NAK_CMD packet.

dl_state <- DL_END

}

EVENT: DEFAULT{

Save current offset as <continue_offset>.

if EVENT is not "data link terminated"

Terminate data link.

dl_state <- UL_UNINIT

}

}

State DL_END{

EVENT: Receive DL_ABORTED_RESP packet. {

dl_state <- DL_CMD_OK.

}

EVENT: Receive DL_COMPLETED_RESP packet. {

Mark file COMPLETE.

dl_state <- DL_CMD_OK.

}

EVENT: DEFAULT{

if EVENT is not "data link terminated"

Terminate data link.

dl_state <- UL_UNINIT

}

}

State DL_ABORT{

EVENT: Receive DATA packet. {

Ignore.

dl_state <- DL_ABORT.

}

EVENT: Receive DATA_END packet. {

dl_state <- DL_END.

}

EVENT: DEFAULT{

if EVENT is not "data link terminated"

Terminate data link.

dl_state <- UL_UNINIT

}

}

State DL_DIR_WAIT{

EVENT: Receive DATA packet. {

Send directory data to user.

dl_state <- DL_DIR_DATA.

}

EVENT: Receive DL_ERROR_RESP packet. {

dl_state <- DL_CMD_OK.

}

EVENT: DEFAULT{

if EVENT is not "data link terminated"

Terminate data link.

dl_state <- UL_UNINIT

}

}

State DL_DIR_DATA{

EVENT: Receive DATA packet. {

Send directory data to user.

dl_state <- DL_DIR_DATA

}

EVENT: Receive DATA_END packet.{

dl_state <- DL_CMD_OK

}

EVENT: DEFAULT{

if EVENT is not "data link terminated"

Terminate data link.

dl_state <- UL_UNINIT

}

}

State DL_SEL{

EVENT: Receive DL_ERROR_RESP packet. {

dl_state := DL_UNINIT

}

EVENT: Receive SELECT_RESP packet. {

dl_state := DL_UNINIT

}

EVENT: DEFAULT{

if EVENT is not "data link terminated"

Terminate data link.

dl_state <- UL_UNINIT

}

}

APPENDIX D - Error Codes

Value Name

1   ER_ILL_FORMED_CMD

2   ER_BAD_CONTINUE

3   ER_SERVER_FSYS

4   ER_NO_SUCH_FILE_NUMBER

5   ER_SELECTION_EMPTY

6   ER_MANDATORY_FIELD_MISSING

7   ER_NO_PFH

8   ER_POORLY_FORMED_SEL

9   ER_ALREADY_LOCKED

10  ER_NO_SUCH_DESTINATION

11  ER_SELECTION_EMPTY

12  ER_FILE_COMPLETE

13  ER_NO_ROOM

14  ER_BAD_HEADER

15  ER_HEADER_CHECK

16  ER_BODY_CHECK

#