FODDRV

Kansas City Tracker Compatible

Rotor Driver for FODTRACK Antenna Interface

Ver 1.1

Mar 10, 1998

Michael Radwick, N8XKZ

FODDRV is a DOS TSR (terminate-and-stay-resident) program which

allows satellite tracking programs such as InstantTrack to

control antenna position using the FODTRACK antenna interface.

FODDRV uses the same tracking program interface as the Kansas

City Tracker and KCT compatible programs.  I have tested the

driver with InstantTrack on DOS 3.2, DOS 5.1 and Windows 3.11.

FODDRV has been tested on a 1MB, 8MHz '286 and on a 4MB, 40MHz

'486 computer.

FODDRV uses your computer's parallel port to communicate with

FODTRACK.  The program does not require any special port

configuration, but the port should not be shared with a

printer.  Nonstandard ports will work fine, you only need to

know the address.  No IRQ is needed.

The latest version of FODDRV is obtained from the AMSAT ftp site

at the following address:

ftp://ftp.amsat.org/amsat/software/PC/rotor-control/

The AMSAT web page maintains a link and brief description of

FODDRV at:

http://www.amsat.org/amsat/ftpsoft.html#pc-rot

The distribution file is named foddrvxx.zip, where "xx" is the

version of the program.  The current release is in the file:

foddrv11.zip

Copyright

---------

Copyright 1998, Michael Radwick.

FODDRV is free for non-commercial use.  If you find the program

useful, then please send a donation to AMSAT.

Installation

------------

Before you start, you should set up and calibrate the FODTRACK

hardware according to the FODTRACK directions.  FODDRV assumes

that the calibration has been done.  Calibration also provides a

sure way of verifying that your hardware is functioning properly.

The distribution file, FODDRV11.ZIP contains the executable and

source files for the rotor control software.  You will need

PKUNZIP to unpack the files.  Assuming that you have downloaded

the ZIP file to your root directory (C:\), use the following

steps to unpack it:

At the DOS prompt (C>) , enter:

C> MKDIR C:\FODDRV

C> CD C:\FODDRV

C> PKUNZIP C:\FODDRV11.ZIP

If you downloaded the ZIP file to a different directory, replace

the "C:\" on the last line with the location of the ZIP file.

After unpacking the ZIP file, you should find the following

files:

KCT.ASM      - Source file for the KCT rotor control hooks

ARGS.ASM     - Generic DOS command line handler

UTIL.ASM     - Some general utility subroutines

FODDRV.DOC   - User Manual/Documentation

BUILD.BAT    - Script to build executable

FODDRV.COM   - Pre-compiled rotor driver

The driver is very simple to install.  Just invoke the driver

from your AUTOEXEC.BAT file.  For example, if you unpack the

driver in the directory C:\FODDRV (as shown above), then add the

following to your AUTOEXEC.BAT file:

C:\FODDRV\FODDRV

Using FODDRV with InstantTrack Accessories

------------------------------------------

If you are using InstantTrack, you may also want to load the

ORBITDRV driver.  ORBITDRV (which comes with InstantTrack)

allows your computer to track a satellite in the background.  I

also use the FLIPANTS program to allow the antennas to be

"flipped over".  I like this because it lets satellites which

would ordinarily pass through the rotor's hard limit to be

tracked without waiting for the antennas to turn 360 degrees.

For everything to work, these programs have to be loaded in the

correct order.  FODDRV is loaded first, followed by FLIPDRV, and

finally ORBITDRV.  The resulting AUTOEXEC.BAT file looks like:

C:\FODDRV\FODDRV

C:\ITTOOLS\FLIPDRV

C:\IT\ORBITDRV

See the References section for information about InstantTrack

and FLIPANTS.

Which Printer Port to Use

-------------------------

By default, the driver loads assuming that the FODTRACK

controller is connected to LPT1.  If you have connected the

controller to a different parallel port, then you can use one of

the following options:

/LPT1     - Directs FODDRV to use LPT1 as the rotor

interface port.  Assumes parallel port at hex

address 378.  This is the default setting.

/LPT2     - Directs FODDRV to use LPT2 as the rotor interface

port.  Assumes parallel port at hex address 278.

/PORT:XXX - Directs FODDRV to use the parallel port at the

specified address.  The address, specified in

hexadecimal, represents the base I/O address of

the port to use as the rotor interface port.

For example, if your computer's parallel port is

addressed at 278, then your AUTOEXEC.BAT file

entry would be:

C:\FODDRV\FODDRV /PORT:278

Adjusting the Antenna Aim

-------------------------

When I installed my antennas, it did not take too long to

discover that my azimuth and elevation were each a "few" degrees

off.  Since I did not wish to climb up and down to mechanically

adjust the antenna, I programmed FODDRV to "fine-tune" the

position for me.

The following command line options allow you to trim your

antennas for maximum accuracy:

/AZ:offset - Offsets the antenna azimuth by the specified

number of degrees. A positive number moves the

antenna to the right, a negative number to the

left.  The offset should be in the range -180 to

+180.  The default value of the offset is 0

(zero).

/EL:offset - Offset the antenna elevation by the specified

number of degrees.  A positive number moves the

antenna up, a negative number down.  The offset

should be in the range -90 to +90.  The default

value of the offset is 0 (zero).

For example:

The Yaesu G-5400 antenna rotors hard stop the azimuth when

the indicator points South.  However, in my setup, when the

indicator is south, the antennas are actually pointing North

(I told you I was a few degrees off)!  So I initially used a

180 degree offset as follows:

C:\FODDRV\FODDRV /LPT2 /AZ:180

This worked pretty well, but I found that the antenna's aim

was still slightly to the left of the satellite.  To move

the offset further to the right requires increasing the

offset. However, an offset of +180 is the maximum positive

offset, so I had to flip the sign (-180), and then add my

offset.  I was only 5 degrees off, so my final configuration

was:

C:\FODDRV\FODDRV /LPT2 /AZ:-175

A couple of notes on usage

--------------------------

There are quite a few different tracking programs out there, but

at home I use Instant Track (IT). When using the satellite

tracker program, azimuth is supplied in the range of 0 - 359

degrees.  The elevation range is -90 to 269 degrees.

One glance at the Yaesu G-5400 will tell you that the rotor's

elevation limits are 0 to 180 degrees.  The FODDRV program

enforces the rotor's limit in this range, after applying the

elevation offset.  FODDRV will ignore elevation requests outside

this range.  FODDRV will, however, still apply the azimuth

setting even if the elevation setting is out of range.  That

way, your antenna's azimuth can be set before the satellite is

above the horizon.

The azimuth value does not appear to be a problem until you

consider the offset provided at the command line.  After adding

offset, the antenna's position could be anywhere between -180

and +539 degrees.  However, FODDRV does not ignore position

settings outside the 0-359 degree range.  Instead, FODDRV will

adjust the commanded position plus or minus 360 degrees in order

to make it fit within the limits of the controller.

One of the features of InstantTrack allows you to display the

status of the rotor control interface.  The status window

displays the current azimuth and elevation settings, and

indicates if the interface is enabled or not.  Ordinarily, you

would expect the azimuth and elevation to show the actual

position of the antenna, even if you moved them manually.

However, the FODTRACK hardware has no means to obtain the

antenna's actual position.  Instead, the values shown in

InstantTrack will reflect the most recently commanded antenna

position.

As noted above, I've only tested FODDRV on relatively slow

PC's.  Since the FODTRACK hardware requires certain minimum I/O

timing, very fast computers may cause interface problems.  A

fast computer could make the timing delays in FODDRV too small

for the FODTRACK hardware to see.  The delays are very easy to

fix, simply add additional 'NOP's after the OUTP instructions in

KCT.ASM and recompile.  If you are unable to do this, let me

know (see the e-mail address below) and I'll try to help.

Another source of potential trouble exists with the DOS interrupt

used by FODDRV.  FODDRV assumes that your tracking program (such

as InstantTrack) uses int 63h to command the antenna rotor.  In

the event (unlikely, in my opinion) that you need to use a

different interrupt, you will need to modify FODDRV and

re-compile.  To do this, edit KCT.ASM, and change the value of

the constant KCTINT to the correct value, then re-assemble.  If

you need to do this, but do not have an assembler, then let me

know.  If there is need, I'll add a command-line option for the

interrupt.

Source Code

-----------

I've included the source code for FODDRV in the ZIP file.

FODDRV is written in assembly language, mainly to keep it

small.  I've provide the script file, BUILD.BAT to show you how

the sources were assembled.  Borland Turbo Assembler 2.0 was

used to build the executable.  No special features of the

assembler were used, so you should be able to build it with

almost any other assembler.

You are encouraged to make changes to the code, but please don't

distribute your version without indicating your modifications.

Don't distribute it at all without my copyright notice and this

.DOC file.

Other References

----------------

Much information on satellites, tracking, and related software

can be found on the AMSAT web page. The AMSAT homepage is:

http://www.amsat.org

FODDRV was built using the program DUMMYKCT as a guide.  DUMMYKCT

provided the programming interface for the KCT, as well as a fine

description of how everything should work.  DUMMYKCT was written

by Paul Williamson, KB5MU, and is available at the following URL:

http://www.amsat.org/amsat/ftpsoft.html

Another handy tool when using InstantTrack is FLIPANTS.  FLIPANTS

was also written by Paul Williamson.  A description of FLIPANTS

and a link to the distribution is available on the web page

shown above.

You may also wish to look at INTSPEC.TXT (supplied with

InstantTrack) for additional info on the programming interface.

INTSPEC.TXT can be downloaded from the AMSAT BBS at the

following site:

ftp://ftp.amsat.org/amsat/software/PC/InstantTrack/intspec.txt

InstantTrack is distributed by AMSAT for a small donation.  More

Information about InstantTrack is on the AMSAT web page shown

below:

http://www.amsat.org/amsat/catalog/software.html

About FODTRACK

--------------

FODTRACK was written by Manfred Mornhiweg, XQ2FOD.  FODTRACK

contains two major components: complete construction details for

hardware to connect your PC to your Yaesu G-5400 Antenna Rotor

Controller; and a satellite tracking program that uses the

interface.  For the home-brewer, you can build FODTRACK in a

single evening for less than $30 in parts!  In my opinion,

FODTRACK is one of the best beginners' projects around.

The only drawback is that the FODTRACK software does not have a

"fancy" graphical display, which shows you where the satellite

is.  With FODDRV, you can use the FODTRACK hardware with the

fancier KCT compatible tracking programs.

More information about FODTRACK (and a link to the latest

distribution) is available through AMSAT at the following URL:

http://www.amsat.org/amsat/ftpsoft.html

Disclaimer

----------

FODDRV is provided without any guarantee at all.  NO WARRANTY is

expressed or implied. However, if you find a problem, please let

me know.  I'll be happy assist you.

Since the source is included, you may be able to fix it yourself.

If so, please forward your changes to me for inclusion in the

next release.  This way, your help will benefit all FODDRV users.

How to contact me

-----------------

I welcome your comments and bug reports.  The fastest way to

reach me is via the Internet.  Send your e-mail to:

n8xkz@amsat.org

Snail-mail works too, at the following address:

Michael Radwick, N8XKZ

7576 Kingsbridge Rd

Canton, MI 48187

Acknowledgments

---------------

My many thanks to Manfred Mornhiweg, XQ2FOD for his easy-to-build

antenna rotor interface.  Thanks to him, I have spent lots of

hours on the satellites without hunting for those elusive Low

Earth Orbit (LEO) birds.

Also thanks to Paul Williamson, KB5MU, whose freeware program

DUMMYKCT provided me with the basis for this software.  Paul

also deserves credit for assisting me with this document, and

with the distribution of FODDRV.

Finally, thanks to Jerry Smyth, N8ULU, who encouraged me to make

this program available to others through AMSAT.