@@@ @   @@@@ @   @     @@@ @@@ @@@@ @   @     @@@@@    @     @

@   @   @  @ @   @     @   @   @  @ @@  @       @      @     @

@@@ @   @  @ @ @ @     @@@ @   @@@@ @ @ @       @       @   @

@ @   @  @ @ @ @       @ @   @  @ @  @@       @        @ @

@@@ @@@ @@@@ @@@@@     @@@ @@@ @  @ @   @       @         @

FOR THE

@@@ @@@@ @  @ @   @ @@@       @@@  @   @@@@ @@@ @@@ @@@ @@@

@   @  @ @  @ @@  @ @  @      @  @ @   @  @ @    @  @   @  @

@@@ @  @ @  @ @ @ @ @  @      @ @  @   @@@@ @@@  @  @@  @@@

@ @  @ @  @ @  @@ @  @      @  @ @   @  @   @  @  @   @  @

@@@ @@@@ @@@@ @   @ @@@       @@@  @@@ @  @ @@@  @  @@@ @   @

Required equipment:

IBM (or compatible) computer, the faster, the better!

Math co-processor not necessary, but also is nice, as the pictures

display faster.

VGA display, capable of 640 x 480 - 256 colors. This requires a VGA

card with at least 512K memory.

Sound Blaster (or compatible) sound card. The program has been

tested with the Sound Blaster V1.5, V2.0, and Pro, as well as Media

Visions Pro Audio Spectrum 16 and Fusion 16 (let me know of others

as you try them!).

This program is designed to send and receive Scottie 1 Slow Scan

pictures that you can find on the ham radio bands. The

frequencies that you will normally find slow scan are:

3.845  MHZ

7.171  MHZ

14.230 MHZ and 14.233 MHZ

21.340 MHZ

28.680 MHZ

The most popular frequency is on 20 meters at 14.230 MHZ, with

14.233 MHZ being used on weekends when there are too many on

14.230.

The following diagrams might help with the electrical hookup. For

receive, the audio from your receiver needs to be connected to the

microphone input on the Sound Blaster. The audio from your receiver

may come from the speaker output. In my case, I used an audio

output from the back of the radio (Icom 730) that is to be used for

RTTY, Slow Scan, packet, etc. The audio out of the back does not

vary with the volume control, so I can keep the audio at a

reasonable level and still copy pictures.

For transmit, take the audio out (one channel, left or right) to

the microphone on your transmitter. Again, I went into the back of

my radio. With the IC730, you have to use an amplified microphone.

Mine just happens to have a level control under the base so the

level into the microphone can be adjusted down to keep the room

noise from interfering with the picture. Use your microphone button

or VOX to key up the transmitter.

Sound Blaster Board V1.5 and V2.0

________________

|

|<---- Sound in from receiver

|

Volume -->|

Control   |

|----> Sound out to your microphone input

|

|

|<---> Midi input/output (not used for this

|      project)

________________|

Sound Blaster Pro

________________

|

|<---- Line in (not used for this project)

|<---- Microphone in - Sound in from receiver

Volume -->|

Control   |

|----> Sound out to your microphone input

|      for transmitting Slow Scan

|

|<---> Midi input/output (not used for this

|      project)

________________|

I did the development on the Sound Blaster, Sound Blaster Pro,

Sound Blaster 16, Galaxy Sound, and even one called the Oki Dokey

Sound Card. Even though most of those cards were capible of

higher sampling rates, I kept the sample rate at 12 KHZ

(the top speed for the Sound Blaster V1.5 and V2.0) so all

Sound Blasters could be used. It gives very acceptable results

at this speed.

Now that the program has been out for a short while, I've received

feedback from others who inform me that it also works on the PAS16

(Pro Audio Vision from Media Vision) and the Fusion 16 (also from

Media Vision) and many other sound cards.

Copy the program to your hard disk (preferably make a separate

subdirectory called SS). Your computer should have several meg

of free space as the .VOC files will take up about 1.5 Meg for each

Scottie picture. You will want to save some of your favorite

pictures in .PCX files so you can recall them later. The reason I

chose the .PCX format is that it takes a lot less space than

.VOC files. The display on your computer needs to be a VGA display

that will work in 640 x 480 - 256 color mode. Some VGA cards do not

have enough memory to do this. If it doesn't work on your system,

I need to know so I can make the necessary changes. When reporting

problems, please make sure to include as much information about

your computer system as possible (autoexec.bat, config.sys, path,

etc), so I can be of help.

The first time that you run the program type "ss". The program will

ask several questions as you start it for the first time.

1. What directory do you want to store the voice files in? I suggest

something like C:\VOC. You MUST create this directory before running

the program.

2. What directory do you want to store the picture files in? I suggest

something like C:\PCX. You MUST create this directory before running

the program.

3. Then it will ask for your callsign or name. If you use your

callsign, it will insert it at the top of the pictures that you

send automatically.

4. It will then ask whether you want to set up the Sound card

automatically or manually. I recommend automatic and only use manual

if auto does not work.

If you choose manual, you will have to choose from a list of sound

cards. If yours is not listed, first try Sound Blaster 8 bit. If that

does not work, try the others one at a time until you find the one

that does. You will also need to specify the sound card address,

interrupt, and DMA channel. Each time, after doing the setup, you

will need to check it by pressing F2 to record a voice file with

sound comming in the mike input. Even if it is static or noise, you

should get speckels on the screen after you press ESC to stop the recording.

The selections that you make are saved in a file called "SS.CFG".

The next time that you run the program, you will not be given the

choices as it knows from the file "SS.CFG" what you selected the

first time. If you need to make changes, you can:

1. Erase the "SS.CFG" file from your hard drive, or

2. Type "ss v" instead of "ss" to start the program. This will

automatically erase "SS.CFG" so you can make your

selections again.

A220 is the base address that you may have set your Sound Blaster

at. Other choices could be A210, A230, A240, A250, and A260. The

Pro only has two choices, A220 and A240.

I7 is the default interrupt of the card. Other choices could be I2, I3,

I5, or I10. I would guess that if you already have the card up and

running with other applications, you only need to find out what

you are using.

D1 is the default DMA channel. I believe that in the Sound Blaster

V1.5 and V2.0, you can only use DMA 1. However, on the PRO you can

use DMA 0, 1, or 3. DMA 1 is the default.

To start the program after it is loaded type "SS" ("SS V" to reselect

the configuration).

Once you have the audio connected, and a station ready to transmit,

pressing F2 will start the receiving process. The sound is stored

in a disk file named "TEMP.VOC". This file is overwritten every

time you press F2. You will not see anything happen until you press

ESC which should be pressed as soon as the audio for the picture

stops. The "TEMP.VOC" file will be a minimum of 1.5 Meg. in size for

one Scottie picture, so you will need to make sure that you have at

least several megabytes free on your hard disk where ss.exe is loaded.

You will learn after using the program that it takes a little bit before

it starts recording, so you need to anticipate the start of the picture

just a little.

If you anticipate too much, you can do the following:

Pressing ESC will stop the recording & start to display whatever

there was to that point

- Pressing ESC again will stop the display

- Pressing F2 will restart the recording.

Once you have a picture on the screen, if you Press F2 again you

will loose the one you have. Menu item F7 allows you to save the

picture as a .PCX file, or you can rename the "TEMP.VOC" file to

another name so when you press F2 again, you do not lose the

picture.

Transmit has been added with the following options.

1. To load a .PCX image that you have created, press F7

followed by F2. Your picture should be previously stored

in C:\PCX (or whatever directory name you specified). Select

the name of the .PCX file of your choice from the menu. The

image will show on the screen. Press ESC to get back to the

main menu. Pressing F8 will convert the picture into a

SEND.VOC file complete with the sync pulses necessary to send

the picture.

2. Pressing F4 will transmit "SEND.VOC", the last picture

converted.

3. Pressing F5 will give you a menu of ".VOC" files that

you have saved. By saving the "TEMP.VOC" file to other

names, you can later re-transmit the pictures you have

received. It's just like a tape recorder!

4. You can see what the SEND.VOC file would look like if

you were to send it by pressing F7 after doing the conversion

in step 1 above. Then press F3 and select SEND.VOC. The

picture will display on the screen, one line at a time as

it would on the receiving end.

Help is built into the program and accessed by pressing F1. This

will give you a second menu of help items. Just press the F key

that will give you the information that you want.

When you send Scottie 1 pictures, it should work fine if the

person on the receiving end will put their unit in SYNC mode.

I have tried it on a ViewPort and with the JVFAX here and it

works fine with the ViewPort without doing anything special, but

with the JVFAX, I have to turn on SYNC MODE to get a straight

picture. I am still doing testing to find out and fine tune

to get it to work with the Robot and Pasokon. The band conditions

have not been the best for testing recently. Hopefully things

will get better.

I hope this program will give you some enjoyment. If it does, I

would also hope that you will register the program for $30.00. I

have spent over 5 years writing this (not every day mind you) and

it was an exercise for me in learning to program in "C". Well, I

guess something must have worked! Anyway, if I get enough response,

I will get to work and enhance the program with the other modes.

Also, I am interested in any ideas you may have to improve the

program or ideas for other programs for the Sound Blaster. I bought

their development kit, and might as well put it to work. So please

send your $30.00 and ideas for enhancement and other programs you

would like to see (how about copying RTTY or PACKET RADIO with the

Sound Blaster ???). Also, anyone sending in the registration with

$30.00 will receive credit towards any major upgrade to the next

higher level.

Speaking of UPGRADES!!!

SLOW SCAN COLOR has also been released as of April 1994 (at the

Dayton Hamvention). Many features were added including:

-The ability to import your own .PCX files and convert to a voice

file for sending

-ZOOM mode to expand the pictures to full screen

-AUTO FREQUENCY TRACKING +/- 150 Hz, a must for the COLOR modes

-SEND and RECEIVE Robot 8,12,24,36 second B&W, Robot 36 & 72 second

COLOR, and SCOTTIE 1 & SCOTTIE 2 COLOR modes and NEW for

October 1995, MARTIN 1 and MARTIN 2 COLOR modes!*

-Larger display area for picture

-Adds callsign to the pictures that you send

-More than 4 times the resolution of Slow Scan II

-SLIDE SHOW for .PCX images that you've saved

-AUTOMATIC MODE CONVERSION for .PCX pictures

-Ability to keep .VOC and .PCX files in seperate directories

-SNOOPER - look at the RBG levels in the picture in waveform format.

You can view Red, Blue, or Green seperately or in combination.

COLOR SLOW SCAN can be purchased for only $99.95!

*Scottie 1 & 2 and Martin 1 & 2 TRANSMIT modes are still being

refined. They do work, but require the receiving end to use the

SYNC mode. Band conditions have not been good (as mentioned above)

so I have not been able to do the testing I would like.

COLOR SLOW SCAN - RECEIVE ONLY

This version has all of the features above with transmit removed.

For the Shortwave Listener, this program will allow you to see

what is going on. It is a lot of fun listening and receiving the

pictures on the Saturday nets (10 a.m. and 12 noon C.S.T).

The RECEIVE ONLY version is available for only $79.95.

Audio Analyzer - Another NEW PROGRAM!

Now you can view the audio signals that you hear on the air! Audio

Analyzer will display the change in frequency in relation to time

so you can "SEE" what Slow Scan, RTTY, Packet Radio, Weather Fax,

and other audio signals 'look like'. The waveforms can be saved to

voice files or viewed REAL TIME! Waveforms saved to .VOC files

can be viewed over and over. If you want finer detail, you can EXPAND

the image or you can SHRINK the image to view more at one time.

The vertical frequency range can be adjusted up or down as needed.

This utility was useful in developing the SLOW SCAN programs and

understanding what is going on. A TIME MEASUREMENT feature allows you

to measure sync pulses, ect., in milliseconds.

AUDIO ANALYZER is available for only $39.95.

Comments or questions can be left for me on Compuserve. Make sure

that you address to Gene Harlan  75211,3622. I am also on

America on Line as well (GeneHarlan).

Questions can be answered evenings at (815) 398-2683. I'm usually

home by 6:00 pm (CST) unless I have meetings, etc. I have enjoyed

helping those that have called to date. All the comments and

compliments were appreciated!!! Compuserve is probably easier and

I try to check it once a day when I'm in town.

We have also installed a FAX line (815) 398-2688. This can be used

as a BBS if you call ahead and give me a few minutes to change it over.

Ads for the COLOR SLOW SCAN are found in most of the major magazines.

You may place an order from 9 a.m. to 9 p.m. CST for credit card

or C.O.D. orders (C.O.D. charge $5.00).

VISA & MASTERCARD and AMERICAN EXPRESS ACCEPTED! Call Today!

Please send the following registration form to:

Harlan Technologies

5931 Alma Dr.

Rockford, Illinois 61108

Registration Information

_________________________________________________________________

______ I have enclosed $30.00 and would like to register the Slow

Scan program. I understand that if I upgrade in the future,

I will get credit towards the upgrade.

______ I have enclosed $79.95 for the Color receive only version.

Please include $5.00 shipping (overseas $10). Illinois

residents include $5.00 tax.

______ I am enclosing $99.95 for the Color transmit and receive

version. Please include $5.00 for shipping (overseas $10).

Illinois residents include $6.25 tax.

_______Audio Analyzer - View the waveforms that you hear for

Slow Scan, RTTY, Amtor, Packet, ect. See the sync pulses and

be able to measure the time in milliseconds. This program

is only $39.95 plus $5.00 shipping ($10 overseas). Illinois

residents include $2.50 tax. Real time samples can be taken

or recorded and played back from .VOC file.

VISA & MASTERCARD and AMERICAN EXPRESS ACCEPTED! Call Today!

Name_________________________________________

Call Sign______________________

Street_________________________________

City___________________________________ State __________

Zip__________   Phone_____________________________

Type/speed of computer __________________________________

Size of hard drive ______________________________________

Type of display card ____________________________________

PLEASE BE SPECIFIC ABOUT YOUR DISPLAY CARD - THIS WILL HELP ME!

Ram - how much do you have ______________________________

Sound Blaster _____ Sound Blaster Pro _____ SB20 _____ SB16______

PRO AUDIO SPECTRUM 16_____ FUSION 16 _____ OTHER __________________

3 1/2 disk _____  5 1/4 disk _____

Other information - ideas - suggestions

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________