FLIPANTS - Flip Antennas on Their Backs

3/21/93 Paul Williamson, KB5MU

Copyright 1993 Paul Williamson.  All rights reserved.

You may use this program for non-commercial purposes.

FlipAnts allows you to track a satellite with your antennas

inverted ("on their backs"), without making any modifications to

your rotor driver program or satellite tracking program.

Why would you want to do that?  Suppose your antennas are installed

with the stops at South.  That is, they rotate from South around

through North and back around to South, but can't track from SouthEast

to SouthWest without going around the long way.  Further suppose that

the satellite pass you're interested in passes South of you (but not

North of you).  You will have to stop in the middle of the pass, and

wait a minute (typically) while the antennas turn 360 degrees.  With

FlipAnts, you can avoid this interruption.  Just turn FlipAnts on

before the beginning of the pass (and make sure your antennas are

capable of turning 180 degrees in elevation).  With the antennas

inverted, the stops are effectively inverted as well.  Stops at

South become stops at North.

This will be especially helpful for low-inclination LEO satellites,

like most Space Shuttle flights.  If your rotor stops are at South

and you're in the Northern hemisphere (or vice versa), you'll have

the problem on every single pass.

On the other hand, with high elliptical orbits like AO-13, you will

occasionally have a pass that passes both North and South of you.

In that case, you'll still have to wait a minute for the rotors to

swing around.  However, with FlipAnts you will at least have your

choice about when this occurs (if you plan ahead).

FlipAnts actually consists of two programs.  FlipDRV is a TSR

(memory resident) program that installs along with your rotor

driver and other resident tracking programs.  FlipDRV actually

takes care of changing the antenna angles computed by your

tracking program so that the rotor driver program sees the

inverted angles.  You will probably install FlipDRV in your

AUTOEXEC.BAT file along with your other tracking TSRs, and

then forget about it.

The other program is called FlipAnts.  You run it from the command

line (or a batch file) to turn FlipDRV on or off.

The interface between FlipDRV and FlipAnts is documented below,

so if you need to control FlipDRV from your own program, you can

do so.

FlipDRV Installation

====================

FlipDRV must be installed AFTER your rotor driver program, RotorDRV

or the Kansas City Tracker program DRV.  If OrbitDRV (the background

tracking program that comes with InstantTrack) is used, it should be

installed AFTER FlipAnts.  If DISPANG (a TSR that displays your

antenna angles in the upper right corner of the text screen) is used,

you may choose to install it before or after FlipDRV.  If you install

DISPANG before FlipDRV, you'll see the flipped angles when FlipDRV is

active.  If you install DISPANG after FlipDRV, you'll always see the

normal angles.

So, a typical installation (in AUTOEXEC.BAT) might look something

like this:

drv/tt=0/ma=0		       <--this loads RotorDRV

osc/hot=$18		       <--this loads the KCT popup

flipdrv                        <--this loads FlipAnts

orbitdrv		       <--this loads OrbitDRV

If you're using a non-default vector for your KCT driver, you can

specify its number on the FLIPANTS command line, in decimal.  Specifying

anything else on the command line will get you a version message and

usage prompt.

CAUTION:

1. Your antennas must be capable of moving 180 degrees in elevation.

2. Your rotor driver program must ignore any angles that your antennas

are not physically capable of.  With FlipDRV on, your rotor driver

will be asked to point to impossible angles (like straight down).

Your driver must know not to try to comply.  The Kansas City Tracker

driver, DRV, is (to the best of my knowledge) safe in this respect.

3. Don't install FlipDRV more than once.  FlipDRV will prevent two

consecutive installations, but it is possible to fool it by

installing another TSR between two FlipDRV installations.  If you

do, you'll get strange results.

NOTICE:

1. FlipDRV won't do anything for you if you operate by preloading the

Kansas City Tracker's tables for a pass.  FlipDRV only works with

the realtime tracking mode used by InstantTrack and OrbitDRV.

FlipAnts Operation

==================

FlipAnts is very simple to use.  To find out the current status, just

say

flipants

To turn FlipDRV on (begin inverted tracking), say

flipants on

To turn FlipDRV off (resume normal tracking), say

flipants off

There's just one complication.  FlipAnts tries to find FlipDRV before

talking to it.  This is always possible if FlipDRV is the last thing

installed, or if everything installed after FlipDRV follows certain

conventions (see the Programming Interface section for details).  If

you have installed something after FlipDRV that doesn't follow the

conventions, FlipAnts will be unable to find FlipDRV.  In that case,

FlipAnts will ask you if you wish to proceed anyway.  If you know

that FlipDRV is installed, you may answer Y.  Otherwise, you should

probably answer N.

Programming Interface

=====================

For a general discussion of how the rotor driver interface works,

see DRVSVC.DOC (from the Kansas City Tracker folks) or INTSPEC.TXT

(which comes with InstantTrack).  The following discussion assumes

a general understanding of the existing interfaces.

FlipDRV adds two functions to the interrupt interface provided by

the RotorDRV program.  One turns inverted mode on or off, and the

other queries its current status.

Function 7F: Set Flip Antennas Status

-------------------------------------

This function enables the user program to set the antenna pointing

mode to normal or inverted.

On Entry:

AH = 7F

AL = 00       for normal antenna tracking

01       for inverted antenna tracking

Other values of AL are reserved.

Function 7E: Get Flip Antennas Status

-------------------------------------

This function enables the user program to determine the current

antenna pointing mode.

On Entry:

AH = 7E

On Exit:

AL = 00       if antenna tracking mode is normal

01       if antenna tracking mode is inverted

Other values of AL are reserved.

Tracking TSR Chaining Convention

--------------------------------

OrbitDRV and all other related TSR programs by KB5MU follow a simple

convention that permits an interested program to follow the chain of

TSR programs attached to the RotorDRV interrupt.  This convention has

two parts:

1. The entry point address of the next TSR in the chain (the one

that was installed immediately before this TSR) is stored at an

address 4 bytes after the entry point of this TSR.  This is

6 bytes before the "RotorDRV" signature that is required of all

TSRs in the chain.

2. The name of this TSR is appended to the "RotorDRV" signature with

a plus sign.  For example, the signature for FlipDRV is

"RotorDRV+FlipAnts".  The signature is terminated by a zero byte.

If you write a TSR that installs in the chain, you are encouraged to

follow this convention.

Source Code

===========

Notice that I've provided you with the source code to both programs.

FlipDRV is written in assembly language, for simplicity and small

resident size.  I used the Microsoft Macro Assembler MASM 5.1, but

just about any assembler should do.  FlipAnts is written in C.

I used the Microsoft C compiler, version 6.00A.  There are a few

non-portable constructs required for this kind of thing, so you

may have to do some work to use a different compiler.

If you make any changes to the code, please don't distribute your

version without clearly labelling it as modified.  Don't distribute

it at all without my copyright notice and this .DOC file.

I encourage you to distribute source code to your programs, too.

Credit and Blame

================

I am always interested in comments and bug reports.  However, this is

just a hobby for me.  I can't promise to provide any support, and

this software is provided with absolutely NO WARRANTY.

I can be reached via:

Internet	pwilliamson@qualcomm.com

or      kb5mu@amsat.org

CompuServe	75265,367

Packet		KB5MU @ KC6NZN.#SOCA.CA.USA.NA

Satellite       ao-16, lo-19, uo-22, kitsat

or by paper mail:

Paul Williamson, KB5MU

5331 Channing Street

San Diego, CA  92117