KANSAS CITY TRACKER/TUNER

Driver Service-Call Functions

Version Dated 3/5/91

*******************************************************************************

*                                                                             *

*       This document is in the public domain and may be freely distributed   *

*       and copied. This document should be copied or distributed intact.     *

*                                                                             *

*       Brooks Van Pelt  KB2CST                  PO Box 1345                  *

*           (609) 751-1018                       Voorhees, NJ  08043          *

*                                                                             *

*******************************************************************************

NOTES:

1) The driver service routines are invoked by performing an INT nn.

Parameters are expected to be passed to the driver in registers.

The default interrupt is INT 63H. While 63h is the recommended

interrupt, this parameter is set at installation and can be varied.

2) DOS normally fills unused user-interrupt-vectors with 0000:0000,

which is not a usable processing address. Application programs that

attempt to invoke the rotor driver when the rotor driver is not

installed will cause the system to immediately crash. Application

programs should test for the presence of the driver before trying to

call a driver interrupt service. The presence of the driver can be

established by using the driver's software interrupt vector number

to obtain through DOS the driver's software interrupt processing

address, adding 10 (0AH) to the displacement, and comparing the

data at this address to this eight-byte field: "RotorDRV". The

compare should be case sensitive. If this data is found the driver

is present.

3) Time/Target table entries that have expired (the date & time of the

entry has passed) are deleted when they arrive at the top of the

Time/Traget table.

4) The service interrupt to be used to access the driver services can be

determined from an application program by searching memory from 40:0

to the application's load point for the this 23-byte string (excluding

the double quotes): "RotorDRV - SVC Vector Z". The string will be

located at displacement 0103H from the base segment address of the

driver. The byte after the last byte of the string will contain the

interrupt vector number (00h to FFh). This method is not recommended

as using this method precludes having more than one copy of the driver

installed and active at a time.

**************************************************************************

Function: Read driver status & binary rotor positions

At Entry: AH=0

At Exit:  AL contains the status bits. The bits are set as follows...

80 - =0 if the PC-card is responding to I/O poll correctly

=1 if the PC-card is absent or not reponding correctly

40 - =0 if the controller is enabled

=1 if the controller is disabled

20 - =1 if the ELEV rotor is stalled & has been disabled

10 - =1 if the AZIM rotor is stalled & has been disabled

08 - =1 if the ELEV rotor is being moved or needs moving

04 - =0 if ELEV rotor is being moved to decrease degree setting

=1 if ELEV rotor is being moved to increase degree setting

02 - =1 if the AZIM rotor is being moved or needs moving

01 - =0 if AZIM rotor is being moved to decrease degree setting

=1 if AZIM rotor is being moved to increase degree setting

BH contains 0 if this is a azimuth-only installation, and 01h if

this is an azimuth-and-elevation installation.

BL contains the signed local-time offset

(UTC + local-time-offset = local time)

CH contains the current binary ELEV rotor position (0-255)

CL contains the current binary AZIM rotor position (0-255)

**************************************************************************

Function: Read current slew setting

At Entry: AH=1

At Exit:  CH contains the ELEV slew binary value (-25 to +25 degrees)

CL contains the AZIM slew binary value (-25 to +25 degrees)

**************************************************************************

Function: Write new slew setting

At Entry: AH=2

CH contains the ELEV slew binary value (-25 to +25 degrees)

CL contains the AZIM slew binary value (-25 to +25 degrees)

Note: If either value is invalid the entire command will

be ignored.

At Exit:  No parameters are returned

**************************************************************************

Function: Read current position and target position

At Entry: AH=3

At Exit:  AX - current ELEV value in degrees (-90 to 269, signed binary)

BX - current AZIM value in degrees (0-359, signed binary)

CX - target ELEV value in degrees (-90 to 269, signed binary)

DX - target AZIM value in degrees (0-359, signed binary)

**************************************************************************

Function: Set Target position

At Entry: AH=4

CX - target ELEV value in degrees (-90 to 269, signed binary)

DX - target AZIM value in degrees (0-359, binary)

Note: If either value is invalid the entire command will

be ignored.

At Exit:  No parameters are returned

**************************************************************************

Function: Clear Time/Target table

At Entry: AH=5

At Exit: No parameters are returned

**************************************************************************

Function: Get Time/Target table data

At Entry: AH=6

CX contains the maximum number of table records to be returned

DX contains the relative table-record number of the first table

entry to be returned. The first entry of the table has a

relative table-record number of zero. If the relative table-

record number is out of range the returned-entry count in

CX will be zero.

ES:[BX] contains the address of where the data should be placed.

At Exit:  AL contains the driver's current table mode...

00=normal

01=repetitive-table-processing mode.

CX contains the number of table entries returned.

DX if the driver is in BBSROTOR mode, DX contains the relative

table entry# of the Time/Target table currently in force.

-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Normal Table Entry Format For Time/Target Read And Write:

+0: Word : Year (the full year, eg, 1987) in binary

+2: Byte : Month in binary (1 to 12)

+3: Byte : Day of month in binary (1 to 31)

+4: Byte : Time hour   in binary (0 to 23) (UTC)

: This byte can be OR'd with 80H to indicate it is the

: first entry of a satellite pass. This is optional.

+5: Byte : Time minute in binary (0 to 59) (UTC)

: This byte can be OR'd with 80H to indicate it is the

: last entry of a satellite pass. This is optional.

+6: Byte : Time second in binary (0 to 59) (UTC)

+7: Word : Target ELEV in signed binary (-90 to 269)

+9: Word : Target AZIM in binary (0 to 359)

+11:BYTEs: 8-byte ASCII satellite identifier. This field MUST be

present and accurate in every table entry for a pass.

+19:L-INT: This field contains the doppler-shifted beacon

frequency of the satellite in hertz. It is a 32-bit

integer whose most significant byte is in the right-

most byte and whose least significant byte is in the

left-most byte.

Table-entry length = 23 bytes

-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Repetitive-table-processing mode Table

Entry Format For Time/Target Read And Write:

+0: Word : FFFFh

+2: Word : Starting minute within cycle period (relative to 0)

+4: Word : Azimuth for this segment (0-359)

+6: Byte : Ten-byte ASCII segment description

+16: Byte : 7-byte unused field

Table-entry length = 23 bytes

**************************************************************************

Function: Add Time/Target table entry

At Entry: AH=7

ES:[BX] points to the Time\Target table image to be added to the

table. This image is copied during the SVC-call

processing and does not need to be maintained after

control is returned to the calling program. The formats

of the table-entry images are described above in the

"GET TIME/TARGET TABLE DATA" section. The driver should

be disabled when adding BBSROTOR-mode table entries.

At Exit:  AL - =0 if the add went ok

=1 if a parameter was invalid

=2 if the table was full & the entry could not be added

**************************************************************************

Function: Read driver's computed date and time

At Entry: AH=8

At Exit:  AX contains the years value in binary (1987 = 07C3h)

BH contains the month in binary (1 to 12)

BL contains the day of month in binary (1 to 31)

CH contains the hours value in binary   (UTC)

CL contains the minutes value in binary (UTC)

DH contains the seconds value in binary (UTC)

DL contains the quarter-secs value in binary (roughly 9/8 fast)

**************************************************************************

Function: Disable/Enable driver movements. The disable function forcibly

stops rotor movements and sets the rotor status as disabled.

The enable function reverses this effect.

At Entry: AH=9

AL=0 for disable, =1 for enable

At Exit:  No parameters are returned.

**************************************************************************

Function: Notify driver of pop-up installation. This command is used by

the TSRs (eg, OSC.COM) during their installation to ensure that

the TSR is only installed once. THIS FUNCTION SHOULD NOT BE USED

BY APPLICATION PROGRAMS AS IT COULD INTERFERE WITH THE CORRECT

INSTALLATION OF THE TSRs.

At Entry: AH=0Ah

BX contains the bit-mask value assigned to this program (eg.,

OSC.COM uses 0001h)

At Exit:  AX contains zero if the calling program has not been loaded

yet, and 0001h if the program has already been loaded.

BX contains the driver's version#.

**************************************************************************

Function: Set ASCII satellite identifier. This command can be used by

application programs to set the satellite identifier displayed

by the OSC status pop-up. This function is entirely optional.

This function is not needed when tabling satellite position

entries, as the table structure has a satellite-id field.

At Entry: AH=0BH

ES:BX - points to the 8-byte satellite-ID text string.

At Exit:  No parameters are returned.

**************************************************************************

Function: This function places the driver into either normal mode or

repetitive-table-processing mode. It also acts to clear the

driver's time/target table, because the table formats for

the various modes are incompatible.

At Entry: AH=0CH

AL=00h - place driver in normal mode.

01h - place the driver into repetitive-table-processing mode.

BX     - Contains the cycle period in minutes when entering

repetitive-table-processing mode. Must be less than

or equal to 10,080 (decimal).

At Exit:  No parameters are returned.

**************************************************************************

Function: This function returns the number of time-target table entries

defined within the driver.

At Entry: AH=0DH

At Exit:  AX contains the number of table entries defined within the driver

**************************************************************************

Function: This function returns the driver's current table-processing mode,

a pointer to the target ID if mode 1, and the current and period

values if mode 2.

At Entry: AH=0EH

At Exit:  AL contains the driver's table mode (0=normal, 1=repeat)

* Mode 1 *

ES:BX - points to the 8-byte ASCII text of the current satellite ID

per the text supplied in the last "First of Pass" record.

* Mode 2 *

BX contains the cycle period in minutes (1-10080)

CX contains the current minute# within the cycle.

**************************************************************************

Function: This function enables or disables the driver's processing of the

Time/Target table during clock interrupts. The function has been

implemented to allow the status pop-up's "spool tbl to disk"

function to work with an unchanging table.

At Entry: AH=0FH

AL=0 means the table-processing function should be disabled.

=1 means the table-processing function should be enabled.

At Exit:  AL =0 if the table function was disabled before this call.

=1 if the table function was enabled  before this call.

**************************************************************************

Function: This function passes a text string to the audible morse-code

sender imbedded in the rotor driver. The text string must

consist of ascii characters with a null (00h) terminator.

Unrecognised characters will be replaced with a space. Multiple

spaces will be replaced with a single space. A 0FFh byte will

generate the rising warble used for AOS notification. A 0FEh byte

will generate the dropping warble used for LOS notification.

At Entry: AH=10H

ES:SI - points to the ASCIIZ string.

At Exit:  No parameters are returned.

**************************************************************************

Function: This function controls the morse-annunicator enablement. It can

also provide the current enablement status.

GET STATUS SubFunction

Entry: AH=11h, AL=0FFH

Exit:  AL=current enablement status (0=dsbl otherwise enbl'd)

DISABLE MORSE SubFunction (also clears any text queued for sending)

Entry: AH=11h, AL=00H

Exit:  AL=prior enablement status (0=dsbl otherwise enbl'd)

ENABLE MORSE WITH OPTIONS SubFunction

Entry: AH=11h, AL=01H to 0FEH (01h is the "PASS&ERR" enablement,

(02h or higher is the "ALL" enablement.)

Exit:  AL=prior enablement status (0=dsbl otherwise enbl'd)

**************************************************************************

Function: This function forces the driver to enable to morse-annunciator

and send the status of the driver in CW. The annunciator

enablement is ignored for this function.

At Entry: AH=12H

ES:SI - points to a 22-byte work area used by the driver

as a work area.

At Exit:  No parameters are returned.

**************************************************************************

Function: This function forces the driver to delete from the Time/Target

table the remainder of the satellite pass. If the first entry in

the table is a start-of-pass record the entire pass is deleted.

At Entry: AH=13H

At Exit:  No parameters are returned.

**************************************************************************

Function: This function controls the rotor-parking enablement. It can

also provide the current parking enablement parameters.

GET PARAMETERS SubFunction

Entry: AH=14h, AL=00H

Exit:  BX=Parking delay time in minutes;

(0 to 60; 0 means disable parking function)

CX=Park-position azimuth (0-359)

DX=Park-position elevation (0-90)

SET PARAMETERS SubFunction

Entry: AH=14h, AL=01H

BX=Parking delay time in minutes;

(0 to 60; 0 means disable parking function)

CX=Park-position azimuth (0-359)

DX=Park-position elevation (0-90)

Exit:  No parameters are returned; Use GET PARAMETERS subfunction

to verify operation of this function. The park timer function

will stopped if it was active when this function was invoked.

STOP PARKING TIMER SubFunction

Entry: AH=14h, AL=02H

Exit:  No parameters are returned.

START PARKING TIMER SubFunction

Entry: AH=14h, AL=03H

Exit:  No parameters are returned.

**************************************************************************

Function: This realtime function passes to the KC Tuner driver the current

doppler shift of the satellite. This signed 32-bit number is

passed as a signed variance relative to an imaginary beacon

frequency of 2**30 (1,073,741,824 decimal or 0x40000000). Note

that this value is negative when the satellite is moving away

from the observer, and positive when the satellite is moving

toward the observer.

At Entry: AH = 15H

BX = Most  significant word of the signed variance

CX = Least significant word of the signed variance

At Exit:  No parameters are returned.

**************************************************************************

Function: This realtime function specifies the satellite name of the

current satellite. This information is used to look up the

satellite's database record. This record provides the nominal

frequencies so that the doppler-shifted offsets can be

calculated.

At Entry: AH = 16h

ES:BX = pointer to a 8-byte, left-justified, space-filled

satellite-name field. This field is not case-sensitive.

If the first character of the satellite-name is a space

the Tuner will change the Tuner window display to show

inactive status, otherwise it will show active status.

At Exit:  No parameters are returned.

**************************************************************************

Function: This function returns the driver's date-sequence display setting.

This setting is used by the OSC and TUNER programs to format the

displayed date into a format appropriate for the user country.

At Entry: AH=17H

At Exit:  AL contains the driver's display setting (0=MM/DD/YY, 1=DD/MM/YY)

**************************************************************************