Vector to Two-Line Elements

(VEC2TLE) Software

User Instructions

Version 9648

29 November 1996

Kenneth J. Ernandes

29-34 170 Street

Flushing, NY 11358

USA

VEC2TLE User Instructions

These are step-by-step user instructions for the VEC2TLE software.  These

instructions are designed primarily for the beginning user.  The instructions

are listed by the different tasks performed by VEC2TLE and are sorted in the

following order, based on what is believed to be high to low incidence of

usage.  The following tasks are covered in these instructions:

A.  SOFTWARE SETUP

B.   AUTOMATIC READ OF STATE VECTORS

C.   KEYBOARD ENTRY OF STATE VECTORS

D.   IMPULSIVE DELTA-VELOCITY MANEUVERS

E.   RE-EPOCH TLE AT THE ASCENDING NODE

F.   IF YOU HAVE TROUBLE

A.  SOFTWARE SETUP

VEC2TLE is designed to reduce operator workload by reducing or eliminating

repetitive tasks.  Important labor-saving features include VEC2TLE's ability

to find and replace Keplerian Elements (both AMSAT and TLE format) as well as

formatted State Vectors that are embedded in text files.  This section

provides procedures for setting up the software for outputting any or all

data formats to the designated output files.  These settings are saved for

future times VEC2TLE is run.  The following steps may be accomplished in any

order and at any time.

A-1  Identify The Two-Line Elements (TLE) Output File

The name of Two-Line Element output file may be specified using the "TLE

Output File" selection under the "File" menu.  This will create a dialog box

listing all files in the VEC2TLE directory with a "*.TLE" extension.  You may

select any file from the list or specify an new filename.  Alternatively, you

may change the default extension to something such as "*.TXT" and select from

the new list.  If the specified file does not exist, VEC2TLE will

automatically create it.

A-2  Identify the AMSAT Keplerian Elements Output File

The name of AMSAT Keplerian Element output file may be specified using the

"AMSAT Output File" selection under the "File" menu.  This will create a

dialog box listing all files in the VEC2TLE directory with a "*.AMS"

extension.  You may select any file from the list or specify an new filename.

Alternatively, you may change the default extension to something such as

"*.TXT" and select from the new list.  If the specified file does not exist,

VEC2TLE will automatically create it.

A-3  Identify the Vector Output File

The name of Vector output file may be specified using the "TLE Output File"

selection under the "File" menu.  This will create a dialog box listing all

files in the VEC2TLE directory with a "*.VIF" extension.  You may select any

file from the list or specify an new filename.  Alternatively, you may change

the default extension to something such as "*.TXT" and select from the new

list.  If the specified file does not exist, VEC2TLE will automatically

create it.

A-4  Specify the Data Output Options

A powerful feature of VEC2TLE is the flexibility of the data output.  The

data output is set by the "Data Output Options" under the "File" menu.  The

three data output formats are:

(a)  Keplerian Two-Line Elements (TLEs)

(b)  AMSAT Keplerian Elements

(c)  Formatted State Vectors

VEC2TLE allows the user to choose any update mode for each of the above data

formats.  The three update modes that may be specified are:

(a)  Replace/Update

(b)  Append

(c)  Disable Output

The Replace/Update is the most commonly-used mode since the old orbital data

is literally replaced with the new.  VEC2TLE searches the applicable output

file for an existing set of formatted data.  If an existing set is found, it

is overwritten with the new data.  If an existing set is not found, VEC2TLE

automatically appends the new data at the end of the file.  In either case,

the user need not be troubled with any "cut and paste" to update the data in

the desired file.

The Append option is used when the user desires to maintain an historical

archive of data.  New sets of data are appended to the end of the output

file.

The Disable Output option is set for output data formats that are not desired

by the user.  This option inhibits output of the particular data format.

B.   AUTOMATIC READ OF STATE VECTORS

Automatic read of formatted state vectors is one of the more popular uses for

VEC2TLE.  The vector format was designed to provide the software with all the

information necessary to produce a complete Keplerian Element set (Two-Line

and/or AMSAT format).  This formatted vector is often embedded in a message

that provides general information.  (An example of a vector message from the

STS-80 Space Shuttle mission is appended to these instructions.)  The VEC2TLE

software will automatically find and read the formatted state vector from the

message.

B-1  Save the Vector Message to a Text File

Select the entire message (or a portion containing the vector) and copy this

to a file.  The message should be saved as a text file with a ".VIF"

extension in the same directory in which VEC2TLE is located -- this makes the

file readily found for read-in by VEC2TLE.

Since it is not possible to provide specific instructions for every text

editor or on-line software package, the following instructions are an example

of how to accomplish this task using typical Microsoft Windows software:

(a)  Choose "Select All" under the "Edit" menu.  (Alternatively, drag the

mouse cursor over a segment of the message completely containing the vector.)

(b)  Copy the vector data to the Windows clipboard -- under the "Edit" menu

select "Copy."

(c)  Create a new text file -- under the "File" menu select "New" (or open

the Windows notepad).  Now paste the contents of the text buffer (the vector

data you copied) into the new file -- under the "Edit" menu select "Paste".

(d)  This new file should IDEALLY be saved in the same directory as VEC2TLE

with a ".VIF" file extension -- under the "File" menu select "Save As."  An

example of a properly formatted ".VIF" file is included with VEC2TLE (file

name is STS-80.VIF).

B-2  Initiate VEC2TLE

(a)  This may be accomplished directly in DOS or launched from Windows if the

program has been installed in the Windows environment.

(b)  Press the "Enter" key or activate the "OK" button with the mouse to

clear the Copyright screen.

(c)  Select the name of the vector file (VIF) you just created -- under the

"Compute TLE" menu select "Read Vector File."  (The Compute TLE menu may be

activated by the mouse or by ALT-C from the keyboard.)  If your file is in

the same directory as VEC2TLE and has a VIF extension, it should appear in

the file list.  You may select your file with the mouse or use the "Tab" and

Up/Down "Arrow" keys to get to your file.  Once your file is selected,

hit the "Enter" key or press the "Open" button with the mouse.  (NOTE:  If

your file is in a different directory, you may choose directory names from

the file list or move up one level in the structure by selecting the ".."

[double period] designator.)

(d)  Once step (c) is accomplished, VEC2TLE should automatically locate the

formatted state vector in your file and read it.  The state vector and its

ancillary information and data will then be displayed in the State Vector

Input dialog box.

(e)  The vector information is likely to be meaningless to most users.

(Details on this information are in the VEC2TLE User Manual.)  If you are

familiar with the data and there's an error (such as a "typo" in the Catalog

Number), this is your opportunity to make the appropriate corrections.  But

if you are like most, just accept the data "as is" by hitting the "Enter" key

or selecting the "OK" button with the mouse.  VEC2TLE will then compute the

Keplerian Elements for you.

(f)  The Keplerian Elements are saved to an output file (default or

previously specified by the user).  OUTPUT.TLE and OUTPUT.AMS are the default

names of the Keplerian element output files.  The names of these files are

maintained in the VEC2TLE configuration file.  It may be changed at any time

by the user -- under the "File" menu select "TLE Output File" -- see section

A.  One or both of these element formats may be read directly by popular

satellite tracking programs.

(g)  NOTE: VEC2TLE is designed to be capable of reading multiple state

vectors [sequentially] from the same VIF file.  This may be accomplished by

repeating steps (c) through (f) for as many vectors as desired or until the

End-Of-File (EOF) is reached.  Since VEC2TLE assumes the VIF file to be the

same, subsequent runs of step (c) will automatically omit the construction of

the VIF file selection dialog box.  (A different VIF file may be selected by

an override -- under the "File" menu select "Vector Input File.")

Sample STS-80 Vector Message from the NASA STSTLE Mailing List

A sample orbital data message from the NASA STSTLE mailing list follows.

This message, originating from STSORBIT PLUS author David H. Ransom, Jr.,

provides the orbital data for revolution 163 of the STS-80 Space Shuttle

mission.  This message is also distributed with VEC2TLE with an STS-80.VIF

filename.  The elements in the message are a handy reference to verify that

VEC2TLE has read the file, found the vector in the file, and updated the

user's AMSAT and/or Two-Line Keplerian element file(s).

Sender: ststle@spacelink.msfc.nasa.gov

Received: from spacelink.msfc.nasa.gov (SPACELINK.MSFC.NASA.GOV

[192.149.89.61]) by hil-img-3.compuserve.com (8.6.10/5.950515)

id XAA22806; Fri, 29 Nov 1996 23:58:18 -0500

Received: from  by spacelink.msfc.nasa.gov (SMI-8.6/SMI-SVR4)

id WAA05795; Fri, 29 Nov 1996 22:58:04 -0600

Date: Fri, 29 Nov 1996 22:58:04 -0600

Message-Id: <Pine.SOL.3.91.961129225120.5619A-100000@spacelink>

Errors-To: rans7500@spacelink.msfc.nasa.gov

Reply-To: ststle@spacelink.msfc.nasa.gov

Originator: ststle@spacelink.msfc.nasa.gov

Sender: ststle@spacelink.msfc.nasa.gov

Precedence: bulk

From: "David H. Ransom" <rans7500@spacelink.msfc.nasa.gov>

To: Multiple recipients of list <ststle@spacelink.msfc.nasa.gov>

Subject: STS-80 TLEs and State Vector (Rev 163)

X-Listprocessor-Version: 6.0c -- ListProcessor by Anastasios Kotsikonas

X-Comment: STS Two Line Elements

MIME-Version: 1.0

X-Disclaimer: Message author is solely responsible for message content.

Organization: NASA Spacelink Teacher Resource Center-a service to educators

STS-80

1 24660U 96065A   96335.12843634  .00000505  00000-0  36422-5 0   354

2 24660  28.4697 113.6679 0009918  24.3381 135.5809 15.72248900  1636

Launch time for STS-80: 19 NOV 96 @ 19:55:47.060 UTC

Earth-Fixed Greenwich State Vector:

Vector format = 117

Satellite Name:         STS-80

Catalog Number:         24660  96065A

Epoch Date/Time:        96335.12843634300

11/30/1996  03:04:56.900 UTC

EFG E:                   -20539482.000000 ft

F:                   7287346.00000000 ft

G:                   3589708.00000000 ft

Edot:                -8777.6200000000 ft/s

Fdot:                -19068.669999999 ft/s

Gdot:                -11317.040000000 ft/s

ndot/2 (drag):              0.00000504920 rev/day^2

nddt/6:                       0.00000E+00 rev/day^3

Bstar:                        3.64216E-06 1/Earth Radii

Elset #:                               35

Rev @ Epoch:              163.44425194215

Keplerian Elements (Amsat format):

Satellite: STS-80

Catalog number: 24660

Epoch time:      96335.12843634

Element set:      35

Inclination:       28.4697 deg

RA of node:       113.6679 deg

Eccentricity:    0.0009918

Arg of perigee:    24.3381 deg

Mean anomaly:     135.5809 deg

Mean motion:   15.72248900 rev/day

Decay rate:    5.04920e-06 rev/day^2

Epoch rev:             163

Checksum:              317

SPACE SHUTTLE ORBITAL DATA:

These 2-line elements have been generated using Ken Ernandes' program

VEC2TLE using real time state vector data supplied by DOD C-Band Radar

Network courtesy Dave Simonson. Note that the epoch of the 2-line

elements is the same as the state vector time.

Drag and B-Star parameters are internally calculated by VEC2TLE using

a drag factor of 0.05. Rev numbers use NASA orbit numbering convention.

Dave Ransom

RPV Astronomy BBS

----------

David H. Ransom, Jr.   Email:rans7500@spacelink.msfc.nasa.gov

C.   KEYBOARD ENTRY OF STATE VECTORS

Keyboard entry of state vectors is required to compute Keplerian elements

from unformatted state vectors.  Unformatted state vectors have been

available for the Space Shuttle from several NASA sources (see Appendix E of

the VEC2TLE User Manual).  Such state vectors are available in a variety of

coordinate systems and units of measure. Therefore, when manually entering

state vectors, you must provide VEC2TLE with the appropriate information

about the vector.

C-1  Set Vector Parameters

The vector parameters that need to be set in VEC2TLE to specify information

about the state vector are: Inertial Reference, Vector Units, Epoch Control,

and Vector Time Reference.  Details on these parameters may be found in the

VEC2TLE User Manual. These parameters may be set as the defaults in the

VEC2TLE.CFG file or temporarily (for the current run of VEC2TLE).  These

parameters are specified under the "Settings" menu. (The Settings menu may be

activated by the mouse or by ALT-S from the keyboard.)

All of the vector parameters are set by "radio buttons."  This provides the

user with a discrete list of legal values from which to choose, ensuring that

only a legal value can be selected.  Once the value(s) is/are set in the

dialog box, the selection(s) can be submitted to VEC2TLE by hitting the

"Enter" key or selecting the "OK" button with the mouse.

(a)  To set these parameters as defaults in the VEC2TLE.CFG file, select

"Defaults" from the Settings menu or "F2."  Otherwise, follow steps (b)

through (e) below.

(b)  The Inertial Reference may be selected from the Settings menu or "F3."

The inertial reference applies ONLY to inertially-referenced state vectors

(i.e., this setting need not be made for Earth-Fixed Greenwich [EFG] / True

of Day Rotating [TDR] state vectors).  VEC2TLE supports True Equator and

Equinox of Date and Mean of 1950 (M50) inertial state vectors.

(c)  The Vector Units may be selected from the Settings menu or "F4."  All

vectors MUST have their proper units identified to VEC2TLE.  VEC2TLE supports

vector units of km,km/s; nm,nm/s; and ft,ft/s.

(d)  The Epoch Control may be selected from the Settings menu or "F5."  Epoch

control is somewhat optional, but setting the epoch of the Keplerian elements

to the time of the state vector is recommended.

C-2  Manually Enter the Vector

(a)  This process begins by the user having VEC2TLE construct the State

Vector Input dialog box -- under the "Compute TLE" menu select "Input State

Vector."  (The Compute TLE menu may be activated by the mouse or by ALT-C

from the keyboard.)

(b)  The user should then enter the appropriate data in each entry position.

The user may elect to have VEC2TLE estimate the following parameters by

entering a "-1" in the respective entry position: Ndot/2 (drag), Nddot/6

(drag), Bstar (drag), and Rev #.

(c)  Some parameters do not have an effect on the Keplerian elements and/or

their computation of the orbital positions and velocities.  The "Satellite

Name or ID" is free-form and may be considered arbitrary.  The five (5) digit

"Catalog Number" will be reflected in the Keplerian elements; leading zeros

need not be entered.  The "International Designator," if not known or

assigned, may be left blank.  The "Element Set #" is more or less arbitrary,

but traditionally follows a numerical sequence.

(d)  The author has the following recommendation for selection of Orbit

Theory: SGP4 is generally recommended for satellites in a Low-Earth Orbit

(Mean Motion greater than 6.4 rev/day); SGP is recommended when Mean Motion

is less than or equal to 6.4 rev/day.  The Mean Motion is the next-to-last

independent number on Line 2 of the TLE (i.e., the line beginning with "2 ").

If the computed Keplerian element set's Mean Motion is inconsistent with the

recommended Orbit Theory, the State Vector Input dialog box may be

reconstructed (i.e., redo steps [a] through [e] ) with the appropriate change

made to the Orbit Theory.  All other parameters in the State Vector Input

dialog box should remain the same.

D.   IMPULSIVE DELTA-VELOCITY MANEUVERS

VEC2TLE provides a somewhat unique capability to estimate the orbit of the

Space Shuttle (or other spacecraft) following an impulsive maneuver.  This

capability provides the ability to immediately compute the post-maneuver

orbit, provided that both a good pre-maneuver TLE and the burn parameters are

available.  This capability was added to VEC2TLE because of the availability

of Space Shuttle "Burn PAD" data on the Mission Control audio of NASA TV.

Estimated Keplerian elements using this feature have proven to be quite

accurate and have tended to agree very well with corresponding post-burn

state vectors.

D-1  Select the Pre-Maneuver Keplerian TLE

(a)  Select the name of the Two-line Element file (TLE) that contains the

pre-maneuver Keplerian TLE -- under the "Compute TLE" menu select "Impulsive

Delta-V."  (The Compute TLE menu may be activated by the mouse or by ALT-C

from the keyboard.)  If your file is in the same directory as VEC2TLE and has

a TLE extension, it should appear in the file list.  You may select your file

with the mouse or use the "Tab" and Up/Down "Arrow" keys to get to your file.

Once your file is selected, hit the "Enter" key or press the "Open" button

with the mouse.  NOTE: If  your file is in a different directory, you may

choose directory names from the file list or move up one level in the

structure by selecting the ".." (double period) designator.

(b)  Ideally, the pre-maneuver TLE should be the first (or only) set of

Keplerian elements in the selected TLE file.  When step (a) is accomplished,

the Two-Line Element Data dialog box is constructed containing the parameters

in the TLE.  If the first TLE is the correct set of Keplerian elements, hit

the "Enter" key or press the "OK" button with the mouse.  Otherwise, hit the

"Esc" key or press the "Cancel" button with the mouse.

(c)  If the desired set of Keplerian elements is NOT the first set in the

file, steps (a) and (b) may be repeated as many times as necessary until

either the correct TLE is found or the End Of File (EOF) is reached.  NOTE

that in subsequent runs of step (a), the selected TLE file is presumed to be

the same.  Therefore, the file selection dialog box will not reappear.  (A

different TLE file may be selected by an override -- under the "File" menu

select "TLE Input File.")

D-2  Specify the Maneuver Burn Parameters

(a)  Once step D-1(c) is accomplished, VEC2TLE will automatically construct

the Impulsive Delta-V Data dialog box.  The user should then enter the

appropriate burn data in each entry position.  Blanks are interpreted as

zeros for both required and optional burn parameters.

(b)  The required burn data parameters are: Ignition Time, Alongtrack

Delta-V, Crosstrack Delta-V, and Radial Delta-V.  The optional burn data

parameters are: Burn Duration, Initial Mass, and Mass Flow Rate.  Assuming

zeros for the optional burn parameters (i.e., instantaneous  delta-V at

ignition with insignificant mass loss) generally provides a reasonable

estimate for the post-maneuver orbit.  For best results, enter accurate

values for the optional burn parameters.

(c)  The mass units used for Initial Mass and Mass Flow Rate are unimportant

as long as they are consistent.  Therefore mass units representing equivalent

Earth weight (such as pounds) are also valid.

E.   RE-EPOCH TLE AT THE ASCENDING NODE

VEC2TLE has the ability to change the epoch time (i.e., reference time) of

the Keplerian elements to the ascending node just prior to an arbitrary epoch

time.  In the past, all Keplerian Two-Line Element sets had the ascending

node as the epoch time.  The main purpose was to simplify orbital revolution

computations.  The implementation of the re-epoch feature in VEC2TLE is to

accommodate the software that requires Keplerian elements to have epoch time

at the ascending node.

NOTE: the Keplerian elements re-epoched at the ascending node are somewhat

less accurate than those with epoch at the time of the state vector; the

degree of difference is directly dependent on the accuracy of the elements'

applicable drag terms.

(a)  Select the name of the Two-Line Element file (TLE) that contains the

desired Keplerian TLE -- under the "Compute TLE" menu select "Re-Epoch TLE at

Asc Node."  (The Compute TLE menu may be activated by the mouse or by ALT-C

from the keyboard.)  If your file is in the same directory as VEC2TLE and has

a TLE extension, it should appear in the file list.  You may select your file

with the mouse or use the "Tab" and Up/Down "Arrow" keys to get to your file.

Once your file is selected, hit the "Enter" key or press the "Open" button

with the mouse.  NOTE: If your file is in a different directory, you may

choose directory names from the file list or move up one level in the

structure by selecting the ".." (double period) designator.

(b)  Ideally, the desired TLE should be the first (or only) set of Keplerian

elements in the selected TLE file.  When step (a) is accomplished, the

Two-Line Element Data dialog box is constructed containing the parameters in

the TLE.  If the first TLE is the correct set of Keplerian elements, hit the

"Enter" key or press the "OK" button with the mouse.  Otherwise, hit the

"Esc" key or press the "Cancel" button with the mouse.

(c)  If the desired set of Keplerian elements is NOT the first set in the

file, steps (a) and (b) may be repeated as many times as necessary until

either the correct TLE is found or the End Of File (EOF) is reached.  NOTE

that in subsequent runs of step (a), the selected TLE file is presumed to be

the same.  Therefore, the file selection dialog  box will not reappear.  (A

different TLE file may be selected by an override -- under the "File" menu

select "TLE Input File.")

(d)  Once the desired set of Keplerian elements are selected, VEC2TLE will

compute the corresponding Keplerian elements with epoch time at the prior

ascending node.  The Two-Line Elements are saved to an output file (default

or previously specified by the user).  OUTPUT.TLE is the name of the default

two-line element output file.  The name of the TLE output file is maintained

in the VEC2TLE configuration file.  It may be changed at any time by the user

-- under the "File" menu select "TLE Ouptut File".  These elements may be

read directly by STSORBIT PLUS or other popular satellite tracking programs.

F.   IF YOU HAVE TROUBLE

VEC2TLE was designed with the intent of having as trouble-free operation as

possible.  Because of the complexity of both VEC2TLE and the large variety of

computer hardware and software configurations, there will inevitably be

situations where things just don't work as expected.  Because of the large

number of potential problems, it's not practical to cover all solutions in

these user instructions.  This section should however, evolve and provide a

list of solutions to some of the more common problems.

Users who experience problems not covered in the list should provide specific

details of the circumstances and the computer's hardware and software

configurations (CONFIG.SYS, AUTOEXEC.BAT, TSR's) to the author.  Instructions

on how to contact the author may be found in Appendix H of  the VEC2TLE User

Manual.

PROBLEM                                  RECOMMENDATION / ACTION

----------------------------------------------------------------------------

Program experiences an abnormal          Use a much smaller drag term or

termination while computing a            drag multiplier (as applicable);

TLE in which a rev estimate was          this occurs mainly with high

requested.                               altitude orbits in which drag is

significantly non-linear.  Alter-

natively, specify the rev value.

Make sure the Vector Time Reference

(UTC or MET) is set appropriately

-- see Settings menu.

Systems hangs and/or program             Delete the VEC2TLE configuration

experiences an abnormal term-            file (VEC2TLE.CFG).  This problem

ination while computing a                has been known to occur when the

Keplerian TLE.                           configuration file is corrupted.

Make sure the Vector Time Reference

(UTC or MET) is set appropriately

-- see Settings menu.

Computed TLE has an extraneous           Delete the extraneous line feed

line feed following the International    at the end of the International

Designator when doing a formatted        Designator in the Vector Input

vector read.                             dialog box or from the TLE.  The

extraneous line file has been

known to be added by some text

editors.

Parameters such a Units or Time          Set the correct reference parameters

Standard (UTC or MET) are incon-         -- under "Settings" menu.  (See pro-

sistent with parameters to be            cedures B-1 for further details.)

manually entered (in procedures          These parameters must be properly set

A through D).                            when manually entering data.