IPS-M Trigonometry Extensions

-----------------------------

by James Miller G3RUH

1997 Jul 20

IPS-M (v1.05 on) may be used for trigonometric operations in the manner of IPS-C

as used in the Amsat P3 satellite flight software, and of IPS-N in the Amsat

ground station software.

The extensions have been implemented by adding three code routines, and

a IPS trigonometry library, supplied in the file "TRIG".

How to load trigonometry routines

---------------------------------

You can use the trigonometry library in two ways:

either: enter the command;    " TRIG " READ

or:     add the library file "Work.TRIG" to your source program

Examples of Use

---------------

1. Calculate cos and sin of 22.5, scaled 10,000.

Enter:     10000 0 4096 T-R  (radius X=10,000, Y=0, T=22.5)

Returns:    9238  3826       (equivalent to 0.9238 and 0.3826 at this radius)

2. Calculate radius and angle of vector <X=3000,Y=4000>

Enter:     3000 4000 R>P

Returns:   5000 9672         (R=5000, angle = ATN(4000/3000) = 53.13)

Code Routines

-------------

The 3 trigonometry code routines are TR-LOOP  RP-LOOP  3V3 .  They are

functionally identical to those in IPS-C3.

TR-LOOP is only called by the library function T-R .  You are unlikely to use

this routine directly.

Entry:  4 items on the stack:  theta, X, Y, 0

Exit:   3 items on the stack:  ~0,    k*X', k*Y'

where k=1.6468.  Routine rotates the vector <X,Y> anti-clockwise through the

angle theta.  The angle theta must lie in the right hand hemisphere; that is,

-90 <= theta <= +90.  The handler T-R ensures this, and is described later.

RP-LOOP is only called by the library function R>P .  You are unlikely to use

this routine directly.

Entry:  4 items on the stack:  theta0, X, Y, 0

Exit:   3 items on the stack:  (theta0 + atn2<X,Y>), k*X', ~0

where k=1.6468.  Routine rotates the vector <X,Y> clockwise through an angle

such as to drive Y' to zero.  The initial vector must lie in the right hand

hemisphere; that is, X must be positive.   The handler R>P ensures this, and is

described later.

RP-LOOP and TR-LOOP are coded using the ARM processor's 32-bit arithmetic for

the Tan/Atn pseudo division and multiplications.  Because of this, these two

code routines are accurate to within +/-0.5 bit at the 16th bit.  The handlers

T-R and R>P degrade this to approx. +/-1 bit.

All library routines, perform "normal" 16-bit operations, and introduce

additional rounding errors typical of that resolution.

IPS Trigonometry

----------------

The following is adapted from the document "Phase III orbit and Attitude

Software" by Karl Meinzer, dated 23.8.79

a) Plane trigonometry

------------------

Classically, all computations can be performed using sine and cosine

functions and their inverse, but these functions tend to be tedious in their

use.  They have the following disadvantages:

* The inverse functions are ambiguous; using them for geometrical problems

usually requires separate resolution of these ambiguities.

* If a sine or cosine carries all the information about an angle, extended

precision is required if the value approaches 1.  With 16-bit angles for

example, 32-bit arguments would be required.

* Sines and cosines force an algebraic analysis of the problem.  Many problems

are much easier to formulate geometrically, though.

These disadvantages stem from the fact that neither a sine or a cosine alone is

sufficient to specify a rotation completely.

A rotation operator may be viewed as a multiplication by exp(i*phi).  Operators

of this kind have not been much used in classical computations because they

involve simultaneous operations on two quantities (2-dimensional vectors).

With most computer languages this approach is not very well supported.  With

IPS the handling of multivariate operators and their results on the stack is

trivial and does not require special considerations.

Two rotation operators are defined.  The approach is identical to that

described by Ball in Byte magazine May 1979 pps.184-194.

The operator T-R (Theta-Rotate) is equivalent to Ball's BB1 and rotates a

vector <X,Y> to the left by theta to produce a new X and Y.

The operator R>P (Rectangular to Polar) takes <X,Y> and rotates it to the right

so as to drive Y to zero.  It thus yields <R,theta>.

With rotators the required processing time is a quadratic function of word

length.  Analysis reveals that 16-bit accuracy is sufficient if the full number

range is employed.  The following number formats were chosen to this end:

Angle representation:

The full circle is defined as 2^16.  Thus 1 count is equal to 0.0055  The

rotators are accurate to 2 or 3 counts or to about 0.01 to 0.02; thus

multiple applications still result in accuracy of order 0.1. The standard

IPS operators addition and subtraction are defined modulo 2^16.  Their

periodicity thus automatically rescales the angles to the correct range; so

angles may be added or subtracted without consideration of overflows.

Two angles are available as constants:

C/4  (a quarter of a circle, 90) has a numerical value of 16384 or #4000.

1RAD (full circle divided by 2 pi) has the value of 10430.

Representation of radius components:

To maintain optimum accuracy the absolute value should be as large as

possible.  Because the cartesian components are signed quantities, 16-bit

numbers have a range or about +/-32,768.  Internally the rotation operators

temporarily multiply the radius components by 1.65.  So to avoid internal

overflows, the absolute value of the radius must not exceed 19898.  Thus it is

recommended that C/4 is used as a general radius value for convenience and

accuracy.

If angles are connected with the radius' length, it is more appropriate to use

1RAD as radius.  C/4 makes full use of the accuracy potential of the rotation

routines; the loss with 1RAD is still very small, and can be ignored.

T-R uses this format on the stack:

entry: X Y theta

exit:  X Y

R>P uses this format on the stack:

entry: X Y

exit:  R Theta

There are two additional maths routines: P* and P/ :

P* multiplies two 16-bit numbers to give a 32-bit result.

P/ divides a 32-bit number by a 16-bit unsigned number to yield a 16-bit

quotient.  All numbers are unsigned.

They are used to build *F (multiply, fractional): a signed 16-bit number is

multiplied by an unsigned 16-bit number and the result is divided by 2^16.  The

second number is thus treated as a fraction, and is for re-scaling purposes.

/N multiplies a signed 16-bit number by 1RAD and then divides the result by

an unsigned 16-bit number yielding a 16-bit result.

CHS changes the sign of a number on the stack.

b) Spherical trigonometry

----------------------

The previously discussed routines are used to build a number of spherical

operators.  The following conventions are used:

Rectangular coordinates:

X Y Z in this order on the stack.  They make up a right-hand screw.

Polar coordinates:

R L B in this order on the stack.  R is the radius, L in the longitude

counted from the 0-meridian to the East and B is the latitude counted

from the equator to the North.

3P>R

converts a point of polar coordinates into rectangular coordinates.

3R>P

converts a point of rectangular coordinates into polar coordinates.

A.B

takes <La,Ba> and <Lb,Bb> and computes their scalar product.

AXB

takes <La,Ba> and <Lb,Bb> and computes their vector product.

It delivers <C/4 x sin(angular spacing), Lc,Bc>  using a right hand screw

convention.

PROJECTION

takes <Lpoint,Boint> and <Lnormal,Bnormal>.

It projects the point onto the plane defined by the normal, and delivers

<C/4 x cos(point with plane), Lproj,Bproj>.  Denoting the point vector P and

the normal vector N, the operation is equivalent to N x P x N.

SK2

computes the second side of a rectangular spherical triangle where the

hypotenuse and the first side are given, ( h,k1 -> k2 ).

This routine could not be solved elegantly by the the approach of viewing all

spherical problems as rotations.  Rather, the classical formula

cos(c)=cos(a)*cos(b) is used.  If c gets small the accuracy is thus limited to

about 0.5.

In addition to these basic functions, a few operators are defined to ease the

handling of typical spherical triples on the stack.

3V3   swaps two triples on the stack

@3    gets a triple from a field address (6 bytes)

!3    puts a triple into a 6-byte field

@L3   get the longitude from a 6-byte field

@LB3  gets longitude and latitude from a 6-byte field

-R3   subtracts two spherical vectors given in rectangular coordinates)

+R3   adds      two spherical vectors given in rectangular coordinates) on the

-P3   subtracts two spherical vectors given in polar coordinates      ) stack

+P3   adds      two spherical vectors given in polar coordinates      )

/A/   returns the absolute value of a 16-bit number on the stack.

Notes

-----

A six byte field contains typically X Y Z or R L B.

L = longitude (from German Laengengrad)

B = latitude  ( ..    ..   Breite     )

P* P/ and CHS are part of IPS-M.