>>>  SATTRAK3.EXE Documentation 06Dec92  <<<

written by Graham Ratcliff VK5AGR

A. Introduction

SATTRAK3.EXE is a command line utility written by N3EMO, VK5AGR and VK5HI which

reads a 'stripped' NASA 2-Line Keplerian Element file and a configuration file

called SATTRAK3.CFG and outputs 3 separate files called KEPS.DAT, ATTS.DAT and

RXTX.DAT which are formatted for easy updating of SATTRAK III by sending them

to SATTRAK III using your favourite communications program.

B. Syntax

The syntax is:

SATTRAK3 <input file>

e.g. SATTRAK3 NASA340.DAT

The input file NASA340.DAT contains the 'stripped' NASA 2-Line Keplerian

Elements as downloaded from your favourite BBS.

The contents of the KEPS.DAT, ATTS.DAT and RXTX.DAT output files are determined

by the contents of the file SATTRAK3.CFG.

Note: SATTRAK3.CFG must be located in the same directory as SATTRAK3.EXE and

that if the <input file> is not in the same directory as SATTRAK3.EXE then the

path must be specified e.g. C:\KEPS\NASA340.DAT

C. The Input File

A typical input file (e.g. NASA363.DAT) would have the following format:

AO-10

1 14129U 83 58  B 92362.21185627 -.00000019  00000-0  99998-4 0  9545

2 14129  27.0008  47.5477 6016919  44.6562 350.5126  2.05877651 71729

UO-11

1 14781U 84 21  B 92360.59548196  .00000821  00000-0  14843-3 0  3913

2 14781  97.8293  28.5366 0011415 186.2765 173.8299 14.68801991471231

RS-10/11

1 18129U 87 54  A 92361.35777432  .00000097  00000-0  99999-4 0  5041

2 18129  82.9260   6.3565 0013367  77.8539 282.4118 13.72304025276147

AO-13

1 19216U 88 51  B 92358.05962542 -.00000248  00000-0  99999-4 0  5496

2 19216  57.6197 342.9243 7279566 303.9629   6.7970  2.09726878 34664

FO-20

1 20480U 90 13  C 92361.70989334 -.00000002  00000-0  24240-4 0  4344

2 20480  99.0629 244.3547 0541766 117.2289 248.5075 12.83215035135199

AO-21

1 21087U 91  6  A 92360.87140303  .00000101  00000-0  99999-4 0  6527

2 21087  82.9429 181.0758 0035919 142.7285 217.6376 13.74504595 95638

RS-12/13

1 21089U 91  7  A 92354.69171526  .00000052  00000-0  48965-4 0  3882

2 21089  82.9226  55.2340 0027976 183.2401 176.8579 13.74009532 93874

UO-14

1 20437U 90  5  B 92360.72761625  .00000218  00000-0  10057-3 0  7047

2 20437  98.6293  80.8484 0011196 335.1745  24.8902 14.29717599152640

AO-16

1 20439U 90  5  D 92357.09600260  .00000215  00000-0  91317-4 0  5383

2 20439  98.6339  77.9827 0011917 349.2408  10.8521 14.29776739152131

DO-17

1 20440U 90  5  E 92356.12237951  .00000227  00000-0  10387-3 0  5404

2 20440  98.6325  77.1792 0011520 352.4649   7.6360 14.29907469152001

WO-18

1 20441U 90  5  F 92355.24997549  .00000231  00000-0  97530-4 0  5426

2 20441  98.6322  76.3511 0011987 355.0839   5.0224 14.29892953151883

LO-19

1 20442U 90  5  G 92357.36201910  .00000215  00000-0  91143-4 0  5394

2 20442  98.6346  78.5897 0012848 347.8812  12.2066 14.29980253152198

UO-22

1 21575U 91 50  B 92360.70382996  .00000300  00000-0  11563-3 0  2387

2 21575  98.4930  73.4736 0008275  93.2092 267.0063 14.36754671 75754

KO-23

1 22077U 92 52  B 92358.01281502 -.00000000  00000-0  99999-4 0   858

2 22077  66.0802 333.0470 0013285 228.6174 131.3703 12.86275083 17188

MIR

1 16609U 86 17  A 92362.59801975  .00017040  00000-0  23447-3 0  7858

2 16609  51.6223   0.8744 0001555 164.7779 195.3104 15.57376113392379

/EX

D. SATTRAK3.CFG File

The configuration file SATTRAK3.CFG is the second file read by SATTRA3.EXE

and has the following format:

SATTRAK3 Configuration File     ; These two header lines must be always be

Copyright AMSAT-Australia 1992  ; present. (Program checks case & syntax).

1               ; SatTrak Number

AO-10           ; Satellite Name

0               ; Alon

0               ; Alat

0               ; AOS Mode (0=NO  1=YES)

1               ; Transponder Mode (0=NO 1=YES)

145.91          ; RX Frequency #1 (MHz)

145.92          ; RX Frequency #2 (MHz)

2               ; RX Mode 1=LSB,2=USB,3=FM,4=FMN

32              ; RX Port Address (in decimal)

10              ; Min. Change in RX Doppler (Hz)

581.004         ; TX Frequency #1 (MHz)

581.004         ; TX Frequency #2 (MHz)

1               ; TX Mode 1=LSB,2=USB,3=FM,4=FMN

34              ; TX Port Address (in decimal)

10              ; Min. Change in TX Doppler (Hz)

#...............;

2               ; SatTrak Number

RS-10/11        ; Satellite Name

0               ; Alon

0               ; Alat

0               ; AOS Mode (0=NO  1=YES)

0               ; Transponder Mode (0=NO 1=YES)

29.357          ; RX Frequency #1 (MHz)

29.407          ; RX Frequency #2 (MHz)

2               ; RX Mode 1=LSB,2=USB,3=FM,4=FMN

0               ; RX Port Address (in decimal)

10              ; Min. Change in RX Doppler (Hz)

145.820         ; TX Frequency #1 (MHz)

145.830         ; TX Frequency #2 (MHz)

1               ; TX Mode 1=LSB,2=USB,3=FM,4=FMN

32              ; TX Port Address (in decimal)

10              ; Min. Change in TX Doppler (Hz)

#...............;

3               ; SatTrak Number

AO-13           ; Satellite Name

210             ; Alon

-5              ; Alat

0               ; AOS Mode (0=NO  1=YES)

1               ; Transponder Mode (0=NO 1=YES)

145.91          ; RX Frequency #1 (MHz)

145.92          ; RX Frequency #2 (MHz)

2               ; RX Mode 1=LSB,2=USB,3=FM,4=FMN

32              ; RX Port Address (in decimal)

10              ; Min. Change in RX Doppler (Hz)

581.398         ; TX Frequency #1 (MHz)

581.398         ; TX Frequency #2 (MHz)

1               ; TX Mode 1=LSB,2=USB,3=FM,4=FMN

34              ; TX Port Address (in decimal)

10              ; Min. Change in TX Doppler (Hz)

#...............;

4               ; SatTrak Number

FO-20           ; Satellite Name

0               ; Alon

0               ; Alat

1               ; AOS Mode (0=NO  1=YES)

0               ; Transponder Mode (0=NO 1=YES)

435.91          ; RX Frequency #1 (MHz)

435.91          ; RX Frequency #2 (MHz)

2               ; RX Mode 1=LSB,2=USB,3=FM,4=FMN

34              ; RX Port Address (in decimal)

10              ; Min. Change in RX Doppler (Hz)

145.89          ; TX Frequency #1 (MHz)

145.91          ; TX Frequency #2 (MHz)

3               ; TX Mode 1=LSB,2=USB,3=FM,4=FMN

32              ; TX Port Address (in decimal)

200             ; Min. Change in TX Doppler (Hz)

#...............;

5               ; SatTrak Number

AO-21           ; Satellite Name

0               ; Alon

0               ; Alat

0               ; AOS Mode (0=NO  1=YES)

0               ; Transponder Mode (0=NO 1=YES)

145.983         ; RX Frequency #1 (MHz)

145.852         ; RX Frequency #2 (MHz)

3               ; RX Mode 1=LSB,2=USB,3=FM,4=FMN

32              ; RX Port Address (in decimal)

200             ; Min. Change in RX Doppler (Hz)

435.016         ; TX Frequency #1 (MHz)

435.016         ; TX Frequency #2 (MHz)

3               ; TX Mode 1=LSB,2=USB,3=FM,4=FMN

34              ; TX Port Address (in decimal)

200             ; Min. Change in TX Doppler (Hz)

#...............;

6               ; SatTrak Number

RS-12/13        ; Satellite Name

0               ; Alon

0               ; Alat

0               ; AOS Mode (0=NO  1=YES)

0               ; Transponder Mode (0=NO 1=YES)

29.408          ; RX Frequency #1 (MHz)

29.408          ; RX Frequency #2 (MHz)

2               ; RX Mode 1=LSB,2=USB,3=FM,4=FMN

0               ; RX Port Address (in decimal)

10              ; Min. Change in RX Doppler (Hz)

145.831         ; TX Frequency #1 (MHz)

145.940         ; TX Frequency #2 (MHz)

2               ; TX Mode 1=LSB,2=USB,3=FM,4=FMN

32              ; TX Port Address (in decimal)

10              ; Min. Change in TX Doppler (Hz)

#...............;

7               ; SatTrak Number

AO-16           ; Satellite Name

0               ; Alon

0               ; Alat

1               ; AOS Mode (0=NO  1=YES)

0               ; Transponder Mode (0=NO 1=YES)

437.026         ; RX Frequency #1 (MHz)

437.051         ; RX Frequency #2 (MHz)

2               ; RX Mode 1=LSB,2=USB,3=FM,4=FMN

34              ; RX Port Address (in decimal)

10              ; Min. Change in RX Doppler (Hz)

145.9           ; TX Frequency #1 (MHz)

145.92          ; TX Frequency #2 (MHz)

3               ; TX Mode 1=LSB,2=USB,3=FM,4=FMN

32              ; TX Port Address (in decimal)

200             ; Min. Change in TX Doppler (Hz)

#...............;

8               ; SatTrak Number

DO-17           ; Satellite Name

0               ; Alon

0               ; Alat

0               ; AOS Mode (0=NO  1=YES)

0               ; Transponder Mode (0=NO 1=YES)

145.825         ; RX Frequency #1 (MHz)

145.824         ; RX Frequency #2 (MHz)

3               ; RX Mode 1=LSB,2=USB,3=FM,4=FMN

32              ; RX Port Address (in decimal)

200             ; Min. Change in RX Doppler (Hz)

0               ; TX Frequency #1 (MHz)

0               ; TX Frequency #2 (MHz)

0               ; TX Mode 1=LSB,2=USB,3=FM,4=FMN

0               ; TX Port Address (in decimal)

0               ; Min. Change in TX Doppler (Hz)

#...............;

9               ; SatTrak Number

WO-18           ; Satellite Name

0               ; Alon

0               ; Alat

1               ; AOS Mode (0=NO  1=YES)

0               ; Transponder Mode (0=NO 1=YES)

437.075         ; RX Frequency #1 (MHz)

437.102         ; RX Frequency #2 (MHz)

2               ; RX Mode 1=LSB,2=USB,3=FM,4=FMN

34              ; RX Port Address (in decimal)

10              ; Min. Change in RX Doppler (Hz)

0               ; TX Frequency #1 (MHz)

0               ; TX Frequency #2 (MHz)

0               ; TX Mode 1=LSB,2=USB,3=FM,4=FMN

0               ; TX Port Address (in decimal)

0               ; Min. Change in TX Doppler (Hz)

#...............;

10              ; SatTrak Number

LO-19           ; Satellite Name

0               ; Alon

0               ; Alat

1               ; AOS Mode (0=NO  1=YES)

0               ; Transponder Mode (0=NO 1=YES)

437.153         ; RX Frequency #1 (MHz)

437.126         ; RX Frequency #2 (MHz)

2               ; RX Mode 1=LSB,2=USB,3=FM,4=FMN

34              ; RX Port Address (in decimal)

10              ; Min. Change in RX Doppler (Hz)

145.88          ; TX Frequency #1 (MHz)

145.9           ; TX Frequency #2 (MHz)

3               ; TX Mode 1=LSB,2=USB,3=FM,4=FMN

32              ; TX Port Address (in decimal)

200             ; Min. Change in TX Doppler (Hz)

#...............;

11              ; SatTrak Number

UO-22           ; Satellite Name

0               ; Alon

0               ; Alat

0               ; AOS Mode (0=NO  1=YES)

0               ; Transponder Mode (0=NO 1=YES)

435.12          ; RX Frequency #1 (MHz)

435.12          ; RX Frequency #2 (MHz)

3               ; RX Mode 1=LSB,2=USB,3=FM,4=FMN

34              ; RX Port Address (in decimal)

200             ; Min. Change in RX Doppler (Hz)

145.905         ; TX Frequency #1 (MHz)

145.975         ; TX Frequency #2 (MHz)

3               ; TX Mode 1=LSB,2=USB,3=FM,4=FMN

32              ; TX Port Address (in decimal)

200             ; Min. Change in TX Doppler (Hz)

#...............;

12              ; SatTrak Number

KO-23           ; Satellite Name

0               ; Alon

0               ; Alat

0               ; AOS Mode (0=NO  1=YES)

0               ; Transponder Mode (0=NO 1=YES)

435.175         ; RX Frequency #1 (MHz)

435.166         ; RX Frequency #2 (MHz)

3               ; RX Mode 1=LSB,2=USB,3=FM,4=FMN

34              ; RX Port Address (in decimal)

200             ; Min. Change in RX Doppler (Hz)

145.897         ; TX Frequency #1 (MHz)

145.85          ; TX Frequency #2 (MHz)

3               ; TX Mode 1=LSB,2=USB,3=FM,4=FMN

32              ; TX Port Address (in decimal)

200             ; Min. Change in TX Doppler (Hz)

#...............; Must be the Last Line of each Entry.

Taking the entry for KO-23 as an example:

12              ; SatTrak Number

KO-23           ; Satellite Name

0               ; Alon

0               ; Alat

0               ; AOS Mode (0=NO  1=YES)

0               ; Transponder Mode (0=NO 1=YES)

435.175         ; RX Frequency #1 (MHz)

435.166         ; RX Frequency #2 (MHz)

3               ; RX Mode 1=LSB,2=USB,3=FM,4=FMN

34              ; RX Port Address (in decimal)

200             ; Min. Change in RX Doppler (Hz)

145.897         ; TX Frequency #1 (MHz)

145.85          ; TX Frequency #2 (MHz)

3               ; TX Mode 1=LSB,2=USB,3=FM,4=FMN

32              ; TX Port Address (in decimal)

200             ; Min. Change in TX Doppler (Hz)

The first line in each satellite entry is the 'SatTrak Number' which is the

'Satellite Select Switch' position that will select this satellite. In the

above example if the 'Satellite Select Switch' is in position 12 then

SATTRAK III will track KO-23. The order of 'SatTrak Number' in the

SATTRAK3.CFG file can be at random.

The second line is the satellite name and for SATTRAK3.EXE to successfully

extract a satellite from the NASA 2-Line Keplerian Element input file the

name of the satellite must be identical to that in the SATTRAK3.CFG or it

will be ignored. UO-22 and uo-22 are not identical nor are AO-16 and PACSAT.

The third line is the Spacecraft ALon (Attitude - Longitude in Bahn

coordinates) and the fourth line is the Spacecraft ALat (Attitude - Latitude

in Bahn coordinates). These two values are used in the SATTRAK III software to

calculate the SQuint Angle (antenna off-pointing) for the Phase III

satellites which are spin-stabilized like AO-13. The values for ALon and

ALat are transmitted by AO-13 on its General Beacon and are published in

the AMSAT News Bulletins.

The fifth line selects AOS (or Continuous) Mode for CAT updating. 0 = NO and

means that the CAT continuously updates both transmit and receive frequencies

when the satellite elevation is greater than the minimum tracking elevation.

1 = YES and means that AOS mode is selected so the CAT stops updating the

receive frequency at AOS i.e. elevation = 0 degrees but continues to update

the transmit frequency. AOS mode is generally used for the 1200 baud PSK

satellites like AO-16, LO-19 and FO-20 which use the up/down tuning of the

PSK modem to keep the receiver on frequency.

The sixth line selects Transponder Mode. 0 = NO and means that the Transponder

mode is not selected in which case the transmit and receive frequencies are

both corrected for doppler. 1 = YES and means that the Transponder Mode is

selected in which case the SATTRAK III will automatically update the transmit

frequency to maintain the desired downlink frequency taking into account the

doppler on the uplink and the downlink. In this mode - the TX frequency that

is entered is the transponder translation constant i.e. the sum of the

uplink and downlink frequencies at the transponder.

The seventh line is the RX Frequency #1 or more correctly the frequency on

which the satellite (beacon or transponder) transmits. In other words the

frequency you would listen to on your receiver assuming that there was no

doppler effect. In the case of KO-23 it transmits on 435.175 MHz FM so to

copy that signal you would tune your RX to 435.175 Mhz FM.

The eighth line is the RX Frequency #2 or more correctly the frequency on

which the satellite (beacon or transponder) transmits. In other words the

frequency you would listen to on your receiver assuming that there was no

doppler effect. In the case of KO-23 it also transmits on 435.166 MHz FM

so to copy that signal you would tune your RX to 435.166 Mhz FM.

The ninth line is the RX mode or more correctly the mode used by the

satellite's (beacon or transponder) transmitter. In other words the mode

you would set on your receiver to decode the signals being transmitted by

the satellite. The choice of modes are 1=LSB, 2=USB, 3=FM and 4=FMN. The

FMN is an extra FM selection that is available on the YASEU FT-736R.

The tenth line is the RX CAT address. For the ICOM brand of radios which use

a Local Area Network type arrangement each radio has a unique address. Enter

the appropriate decimal number from the table below for the RX CAT address.

Radio      Hex Address  Decimal Address

IC271A/E/H      20H            32

IC471A/E/H      22H            34

IC1271A/E       24H            36

IC275A/E/H      10H            16

IC475A/E/H      14H            20

IC1275A/E       18H            24

IC970A/E/H      2EH            46

For the Yaesu radios (FT736 and FRG9600) the RX CAT address can be set to

any value. We suggest you enter zero.

For Kenwood radios (see SatTrak III user and assembly manuals for additional

information) one enters either a zero, one or two for the address. Kenwood

radios require an additional hardware interface which is described in an

appendix to the SatTrak III Assembly Manual. It has two outputs labeled

Kenwood-1 and Kenwood-2. If you use a TS-790 connect it to Kenwood-1 and enter

0 for the RX CAT address. For the TS-711/811 separate radios you enter a 1 or

2 depending on the connection. Let us use the KO-23 example again. The satellite

transmits on 435.175 MHz. So you will receive on the TS-811. If we connect it

to the Kenwood-2 port of the interface we will enter a 2 for the RX CAT address.

We could just as well connect the TS-811 to the Kenwood-1 port. In that case we

would enter a 1 for the RX CAT address.

The eleventh line is the Minimum Change that must occur in the RX frequency

before the SATTRAK sends a RX Frequency update to your receiver. For example

if the Min. Change in RX Doppler has a value of 200 Hertz then if the last

time the frequency was updated was 435.125485 MHz then until the doppler

corrected RX Frequency is greater than 435.125685 MHz or less than

435.125285 MHz then the RX Frequency will not be updated.

Lines 12,13,14,15 and 16 refer to the equivalent settings for your transmitter,

in other words to 'match' the receiver on the satellite. In the case of KO-23 it

has a receiver on 145.897 and 145.85 MHz therefore you transmit on 145.897 or

145.85 Mhz. For the TX CAT address enter data similar to that described for the

RX CAT address, again depending on the brand of radio that is being controlled.

That is, for the FT-736 enter 0 for the TX CAT address. For the TS-790 also

enter 0. For the ICOM radios and the Kenwood separates enter the appropriate

address following the example described above for the RX CAT address.

E. The Output Files

As indicated earlier, SATTRAK3.EXE creates 3 new specially formatted ASCII data

files named:

KEPS.DAT  -  Updated Keplerian Elements

ATTS.DAT  -  Spacecraft Attitude Data

RXTX.DAT  -  Rx and Tx Settings

These 3 files can be input (transmitted) to SATTRAK III to automatically update

or change all of these parameters.

The steps are as follows:

1. Using your favourite communications package to 'talk' to SATTRAK III

switch on the SATTRAK III and press <Enter> to get the 'Main Menu'.

2. Make sure that your communications package has the ability to perform

plain ASCII uploads using a line pacing character. Set the line pacing

character to a question mark '?'. Some programs may use a time delay between

lines transmitted instead of the pacing character. Experiment with different

times. We have found that 50/60 second works well.

3. Follow the procedure to perform an ASCII upload of any one of the Output

Files KEPS.DAT, ATTS.DAT or RXTX.DAT and you should see on the screen

the data being automatically updated one line at a time as if you were

manually typing the data in on your keyboard - except much faster.

F. Support.

Contact: Graham Ratcliff VK5AGR

AMSAT-Australia National Coordinator

G.P.O. Box 2141

Adelaide, S.A. 5001

Phone: (08)2975104 between 0900 and 1100utc

Ron Long W8GUS                                                                                                                                                                                                                                                 Kiron Corporation

1516 Essex Road

Columbus, OH 43221 USA

Phone: 614-481-0541.

<<<< end >>>